# A Design Flow and Tool for Avoiding Asymmetric Aging

Freddy Gabbay,1 Avi Mendelson,2 Basel Salameh,2 and Majd Ganaiem2

| Ruppin Academic Center1 freddyg@ruppin.ac.il | Technion – Israel Institute of Technology2 {avi.mendelson, basel238, majd.ga}@technion.ac.il |
| --- | --- |

Abstract — The latest process technologies have become susceptible to asymmetric aging, which causes the timing of logical elements to degrade at unequal rates over time, resulting in reliability concerns. Most existing tools can handle asymmetric aging in relatively small circuitry and rely on a physical design approach. This paper introduces a design flow and tool to minimize asymmetric aging in data-path structures. The tool can be integrated into standard design flows of large-scale circuits. In addition, it automatically embeds a special circuitry in the design to eliminate asymmetric aging. Experimental analysis reveals the approach eliminates reliability concerns while introducing minor overhead.

Keywords — Asymmetric Aging, Reliability, Bias Temperature Instability, Electronic Design Automation, EDA

# INTRODUCTION

Advanced VLSI process nodes have become highly susceptible to reliability concerns. At the same time, new application areas such as data centers, medical appliances, and automotive systems are imposing severe reliability requirements on integrated circuits (ICs) [1, 2]. The semiconductors industry has identified transistor aging as one of the major reliability concerns. Two physical mechanisms induce transistor aging: hot carrier injection (HCI) and bias temperature instability (BTI) [2–4], both of which lead to transistor failure and degraded performance. The common approach to handling transistor aging is to apply extra timing margins to the clock-cycle time. When timing degradation due to aging is symmetric, this can be a suitable solution; however, many logical elements may undergo asymmetric aging, whereby logical paths may experience critical timing violations even if the delay shift is relatively small.

BTI, which may arise when a constant voltage is applied to transistor gates, is the main cause of asymmetric aging. Prior studies [2] report that, for BTI to induce transistor timing degradation, such voltages must be applied for durations ranging from seconds to several weeks. This observation is disturbing because constant logical values, when applied to any logical element, may result in severe reliability issues. The common techniques [5–17] used to handle asymmetric aging rely on complex physical design solutions that may involve nonscalable analysis and simulation methods. Common electronic design automation (EDA) tools have limited capabilities to offer adequate solutions, which exacerbates the problem.

In our previous work [6], we introduced an automated EDA tool that helps avoid asymmetric aging in data-path designs. The tool analyzes the design and identifies elements that are suspectable to asymmetric aging. It generates special circuitry that is embedded in the design to avoid asymmetric aging while the device is in mission mode. This paper extends that work and enhances the tool to support automated engineering change order (ECO) flow (Section III). In addition, we extend our experimental analysis and examine new data-path modules (Section IV).

After a brief review of asymmetric aging and prior studies, we illustrate how the proposed tool automatically analyzes data-path structures, identifies elements suspectable to asymmetric aging, and generates a special circuitry to avoid asymmetric aging. Our timing simulations combined with aging models indicate that the tool efficiently eliminates asymmetric delay shift and thereby avoids BTI reliability issues. Experimental results show that the design flow incurs negligible power and area overhead and that it can be straightforwardly integrated as part of standard design flows of large-scale ICs.

# Asymmetric Aging

With 28 nm and smaller process technology nodes, ICs grow increasingly vulnerable to reliability issues. Reliability issues introduce major challenges to EDA tools and ICs mainly because they involve physical phenomena that require complex simulation and analysis for various combinations of temperature, voltage, and process corners. The semiconductor industry has significantly invested in efforts to cope with reliability issues, which mainly consist of attempting to enhance physical implementation flows. However, the effectiveness of such flows is limited, primarily due to their small scale and because they involve substantial design efforts.

## Asymmetric Transistor Aging

Transistor aging is the process whereby silicon transistors develop faults in their circuitry over time [2]. This degradation is induced by two physical mechanisms: HCI and BTI. When HCI is involved, excessive energy causes charge carriers from the source-drain current to be trapped at the gate oxide. For BTI, the charge carriers are trapped whenever a constant voltage is applied to the transistor gate, but no current flow is involved in this case. Both HCI and BTI lead to an increase in transistor threshold voltage, which degrades the transistor speed and mandates a higher voltage to switch on the transistor. This imposes extra timing margins in the clock-cycle time to allow ICs to continue operating reliably throughout their lifecycle. Various approaches have been proposed to handle transistor aging, including physical-design- and circuit-based solutions [7].

When the aging degradation of transistors is not uniformly distributed, the problem becomes even more complicated. This may happen when p- and n-type devices age unequally, in which case the rising and falling transients incur asymmetric delay shifts. The problem can also occur when the timing degradation between different logical paths becomes asymmetric [5]. This may lead to severe setup and hold timing violations, which cannot be identified by conventional timing analysis tools. This phenomenon, referred to as “asymmetric aging,” is a major reliability concern in ICs.

Asymmetric aging is induced by long periods of unequal static stress applied to logic elements in different logical paths, which may range from tens of seconds to several weeks [2]. As the main contributor to asymmetric aging, BTI can degrade the speed of both p-type (PBTI) and n-type (NBTI) devices. The NBTI’s impact is more significant, although in advanced process nodes, PBTI’s impact is also considerable.

Asymmetric aging introduces major challenges due to the complex modeling and analysis required to eliminate its impact in large-scale circuits. In particular, timing-verification tools that model BTI effects are nontrivial because they depend on the functional modes of operation throughout the IC’s lifetime. From an architectural point of view, asymmetric aging in many cases is a result of dynamic power-saving techniques that impose static states on logical circuits.

## Prior Studies

Many prior works approached asymmetric aging from the physical design point of view [7]. Such an approach is not straightforward, because the process of simulating, analyzing, and fixing asymmetric aging issues in large-scale circuits is highly complex, and only a few EDA tools have such a limited capability. To cope with this problem, traditional approaches rely on taking margins in timing closures that would consider the asymmetric aging effect. This necessitates a highly complex analysis and, in many cases, resulted in overdesign. Other studies attempted to model and predict the degradation as a result of NBTI [2] and suggested solutions such as transistor sizing, VDD tuning, duty cycle reduction, and decreasing the transistor channel length.

Only a limited number of works have suggested architectural solutions for asymmetric aging. Firouzi et al. suggested a NOP (no operation) instruction insertion to reduce the impact of NBTI on microprocessors [8]. Abbas et al. suggested running antiaging programs when the processor is not utilized [9]. This technique was efficient, but it required a complex analysis of the critical paths and the requisite antiaging values. Other studies have proposed solutions for asymmetric aging in memory systems [4].

# Design Flow and Tool for Asymmetric Aging Avoidance

The proposed new design flow and tool are based on our prior study [5] in which we observed that data-path circuits may be susceptible to asymmetric aging under different workloads. Past experiments suggested that microprocessor execution units may incur very long static BTI stress. For example, for integer workloads the utilization of FP execution units is extremely low, resulting in excessive BTI stress. In our earlier work [5], we introduced a novel scheme to mitigate BTI stress over FP adder. The proposed scheme used a pseudorandom sequence bit (PRBS) generator [5] that generates pseudorandom patterns into the data path of the FP adder unit to prevent extended periods of constant stress.

This study generalizes that prior work and extends it to automatically handle any data-path logical structure. We introduce a full design flow and an automated EDA tool to minimize the asymmetric aging effect in data-path structures. After first presenting our design flow, we describe the implementation details of our tool.

## Asymmetric Aging Avoidance Design Flow

Figure 1 (a) depicts the proposed design flow, which consists of two iterative phases. In the first phase, the analysis is done at the register-transfer level (RTL), and the second phase is performed on the synthesized design at the gate level. In the RTL phase, our tool runs on the original design, and the run setting is specified through a configuration file, which we will describe later. The tool automatically generates two modules: a testbench module and a synthesizable module, which Figure 1 (b) shows. The light and dark gray colors in Figure 1 (b) indicate all the logical components that are automatically added by the tool. For the synthesizable design module, the tool generates a top-level wrapper that instantiates the original data path, a PRBS generator, and a multiplexor. The PRBS generator, which is activated by a slow frequency clock, generates pseudorandom patterns that are fed into the module through a multiplexor to prevent extended periods of constant stress when the module is idle. The multiplexor arbitrates between the functional inputs and the PRBS outputs. The slow clock frequency can be on the order of megahertz or lower to minimize dynamic power overhead.

The testbench is used to quantitively measure the effectiveness of the asymmetric aging avoidance circuitry. The testbench module instantiates the synthesizable module with the clock generators and a set of signal-probability counters. The tool automatically maps all the nets in all the hierarchies of the original design, and it associates each net with an individual counter. Through the RTL simulation, the counters measure the number of clock cycles in which the corresponding net is in the logical state of 1. We calculate the signal probability 1, denoted as SP(1), of every net in the synthesizable design by dividing the corresponding counter value by the number of simulated clock cycles.

In the next step of our design flow, the tool invokes an RTL simulation of the testbench, and once completed, it dumps the signal-probability counter values and the corresponding net names into a temporary file. The file is postprocessed by the tool, and a report, which summarizes the SP(1) of every net in the design, is generated. The report highlights the nets with excessive BTI stress that could not be mitigated by the asymmetric aging avoidance circuitry. If some of these cases can be fixed manually at the RTL, the process will repeat itself until there are no manual fixes left. Figure 1 (c) illustrates an example of such a manual fix where one of the multiplexor inputs is connected to logical 0, inducing a constant BTI stress. This can be simply eliminated by using the equivalent circuit shown in Figure 1 (c).

(a)

(b)

(c)

Figure 1 – (a) Asymmetric aging avoidance design flow, (b) an automatically generated design with asymmetric aging avoidance circuitry, and (c) manual fix at the RTL.

The second phase of our flow runs at the gate level. The synthesizable module only and the asymmetric aging avoidance circuitry are synthesized, and the gate-level netlist is presented to the tool as an input. The tool automatically generates a testbench module, as Figure 1 (b) shows. Next, the tool automatically analyzes the gate-level netlist and uniquely associates every net to an individual signal-probability counter. In the next step, the tool invokes a gate-level simulation, and once completed, all counter values are dumped into a temporary file and a signal-probability report is presented to the designer. Because the PRBS pattern may not be able to propagate to all the nets, a step of incremental fixes may be needed to handle these nets. Incremental fixes may be done manually or can be automatically performed by the tools.

The incremental-fix procedure is done by the tool as an ECO procedure on the gate-level netlist. The tool provides the designer with two options (which can be combined) to specify the required fixes. In the first option, the designer provides a list of nets for the tool to fix. In the second option, the designer provides two SP(1) thresholds: threshold-high (thH) and threshold-low (thL). Given the two thresholds, the tool fixes all nets with SP(1) in the range 0 to thL and thH to 1. Identifying the nets that require incremental fixing involves running a timing verification with aging models in conjunction with the SP(1) values from the flow. All the nets from the violating paths are candidates for incremental fixes.

Figure 2 illustrates the fixes that can be done by the tool. The tool detects all the nets to be fixed and inserts a multiplexor to drive the original net’s input. One of the inputs of the multiplexer is connected to the original driver cell, whereas the second input is connected to one of the outputs of the PRBS generators. The multiplexor selector signal is controlled by the slow clock circuitry. With this method, the tool ensures that the pseudorandom pattern propagates through a selective number of nets in the design. Note that the fix process is fully controlled by the designer. The designer can adjust for the overall design considerations and initiates the needed tradeoffs between the required number of fixes and the power and area overhead associated with the incremental fixes (also included by the tool’s report).

Figure 2 – Automatic incremental fix.

## Asymmetric Aging Avoidance Tool Architecture

Figure 3 illustrates a block diagram of our tool. The tool, which we designed in the Tcl language, includes the following software modules: configuration-file parser, hardware description language (HDL) generation engine, signal-probability analyzer, incremental-fix engine, and report generator. The configuration-file parser reads the configuration file supplied by the designer (see Figure 3). The HDL generation engine creates a new top-level wrapper for the synthesizable design and a testbench module. The HDL generation engine invokes the simulation, and once completed, a dump file of the SP(1) information is created. The dump file is analyzed, and the information is provided to the report generator. The report generator can generate the reports described in Figure 3. Finally, the incremental-fix engine identifies all nets that require fixes and inserts the needed incremental fixes.

Figure 3 – Asymmetric aging avoidance tool block diagram.

# Experimental Results

As part of our experimental analysis, we examine the effectiveness of the full design flow presented in Figure 1 (a) on integer ALU, FP adder, FP multiplier, FP divider, and FP round logic. All modules were obtained from the OpenCores repository. Our analysis is performed on 28 nm process technology using the Synopsys Digital Cell Libraries [10]. The synthesis tool (Synopsys Design Compiler Q-2019.12-SP1) ran with a core voltage of 1.05 V, a clock frequency of 420 MHz, and a process corner SS_1p05_125C (Slow-Slow corner). We used the PRBS9 generator [6] and a simulation runtime of 1 million clock cycles.

## Signal-Probability Experimental Analysis

The first part of our experimental analysis examines the SP(1) distribution. Initially, we ran the tool at the RTL and generated histogram reports of the signal probability for each module. Next, we synthesized every module with the asymmetric aging avoidance circuitry. Finally, we ran the tool on the gate-level netlist and generated SP(1) histograms. Figure 4 presents the histograms for both the RTL and gate level. As previously indicated, designs that allow static BTI stress for long periods may incur serious reliability issues due to asymmetric aging. For example, microprocessors that run integer applications do not use FP execution units. In some cases, even the available integer ALUs cannot be fully utilized. Prior studies [5, 6] show that, in such cases, approximately 50% of the nets are static logical 1, while the rest are static logical 0. Figure 5 shows that the proposed tool and circuitry can remove the continuous BTI stress from the majority of nets. The FPU adder, FPU multiplier, and ALU all have a SP(1) in the range of 30% to 70% for the majority of nets. The FPU divider and FPU round modules have an even tighter distribution, and the majority of their nets have a SP(1) in the range of 40% to 60%. The histograms show that a smaller portion of nets cannot be toggled effectively and remain static through most of the simulations. We have identified that this behavior is due to (1) constant values in the design and (2) logical shifters in the integer ALU and in the FPU divider that involve many nets with constant zero padding. Such nets are easily fixed by running the proposed tool in incremental-fix mode. This may only be necessary for a small number of nets because such nets are not typically on the critical path in an asymmetrically aged design.

Figure 4 – SP(1) histograms for execution units.

## Power and Area Overhead Analysis

As part of our experimental analysis, we integrated all modules and wrapped them together in a top-level module to represent an execution unit that combines multiple processing elements. We synthesized the combined execution unit using the process technology parameters we described earlier. We ran the asymmetric aging avoidance flow on the combined gate-level netlist. The total area and power of all modules was 0.1455 mm2 and 0.3 mW, respectively. The asymmetric aging avoidance circuitry area and power overhead was 0.0022 mm2 and 0.3 mW, respectively (1.8% and 2.7%, respectively).

## Timing Analysis Based on Aging Models

In the last part of our experimental analysis, we performed a timing analysis based on aging models to examine the improvement in reliability gained by the proposed design flow. In this method, we extended the conventional timing models with BTI-aging models in which the rising cell delays are derated by their corresponding NBTI degradation factors (as a function of the SP), whereas the falling delays remain unchanged. The derate factors for the aging libraries were generated by using SPICE simulations with the nominal Vth values replaced by aged Vth values that correspond to the lifetime and SP. The Vth degradation model is based on the reaction-diffusion model, which is the most widely accepted model for BTI aging [6, 11].

Table 1 summarizes our timing analysis comparison of fresh, aged, and asymmetric-aging-aware designs. The results show that all modules incur setup violations when asymmetric aging is considered. In addition, the FP adder, FP round, and FP divider also experience hold violations. Note that, even if the integer ALU still meets the hold constraints, the positive slack decreases and thus the design overhead becomes minimal. Table 1 also shows that the tool significantly reduces the timing violations of the asymmetrically aged designs. The FP divider, FP round logic, and integer ALU are clean from any timing violations. Both FP multiplier and FP adder exhibited a small number of setup and hold violations that were cleaned by applying the incremental-fix flow. In both modules, the number of paths involved was small, so the power and area overhead were negligible.

Table 1

setup and HOLD VIOLATED Endpoint Paths for fresh, aged, and asymmetric-aware desIgns.

|  |  | Setup |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fresh |  | Aged | Asym. Aging avoidance tool |  | Incremental-fix flow |  |  |
| FP Add. | 0/687 |  | 4/687 | 1/687 |  | 0/687 |  |  |
| FP Mul. | 0/1803 |  | 12/1803 | 3/1803 |  | 0/1803 |  |  |
| FP Div. | 0/992 |  | 4/992 | 0/992 |  | 0/992 |  |  |
| FP Rou. | 0/322 |  | 8/322 | 0/322 |  | 0/322 |  |  |
| Int. ALU | 0/134 |  | 2/134 | 0/134 |  | 0/134 |  |  |
| Hold |  |  |  |  |  |  |  |  |
|  | Fresh |  | Aged | Asym. Aging avoidance tool |  | Incremental-fix flow |  |  |
| FP Add. | 0/687 |  | 59/687 | 3/687 |  | 0/687 |  |  |
| FP Mul. | 0/803 |  | 68/803 | 4/1803 |  | 0/1803 |  |  |
| FP Div. | 0/992 |  | 116/992 | 0/992 |  | 0/992 |  |  |
| FP Rou. | 0/322 |  | 115/322 | 0/322 |  | 0/322 |  |  |
| Int. ALU | 0/134 |  | 0/134 | 0/134 |  | 0/134 |  |  |

# Conclusions

IC reliability is a crucial requirement that has been highly challenged by advanced process technologies and new computation-intensive applications. Recent advanced process nodes have become highly susceptible to asymmetric aging that can cause critical timing violations in ICs and overall system failure. In this paper, we introduce a novel tool and design flow to mitigate the asymmetric aging effect. Further studies on EDA flows to cope with asymmetric aging are highly encouraged because the effect of BTI and HCI are further intensified in advanced process nodes. This is a major challenge for both industry and research communities in the effort to identify practical solutions that allow the development of future reliable ICs. A possible future direction of research is to further extend the proposed tool and design flow to control path modules, memory systems, and domain-specific accelerators.

References

Failure Mechanism Based Stress Test Qualification for Integrated Circuit. Automotive Electronics Council, Component Technical Committee – AEC – Q100 – REV-G standard.

S. Bharadwaj, W. Wang, R. Vattikonda, Y. Cao, and S. Vrudhula, “Predictive modeling of the NBTI effect for reliable design,” in Proc. Custom Integrated Circuits Conf., Sep. 2006, pp. 189–192.

A. Campos-Cruz, G. Espinosa-Flores-Verdad, A. Torres-Jacome, and E. Tlelo-Cuautle, On the prediction of the threshold voltage degradation in CMOS technology due to bias-temperature instability. Electronics, vol. 7, no. 12, pp. 427 2018, 7, 427.

A. Calimera, M. Loghi, E. Macii, and M. Poncino, “Aging effects of leakage optimizations for caches,” in Proc. IEEE Great Lakes Symp. VLSI, May 2010, pp. 95–98.

F. Gabbay, A. Mendelson. Asymmetric Aging Effect on Modern Microprocessors. Microelectronics Reliability. Vol. 119, April 2021, 114090.

F. Gabbay, A. Mendelson, B. Salameh and M. Ganaiem Asymmetric Aging Avoidance EDA Tool. 34th SBC/SBMicro/IEEE/ACM Symposium on Integrated Circuits and Systems Design (SBCCI), 2021, pp. 1-6, doi: 10.1109/SBCCI53441.2021.9529984.

M. Taddiken, N. Hellwege, Nils Heidmann, Dagmar Peters-Drolshagen and Steffen Paul. Analysis of aging effects - From transistor to system level. Microelectronics Reliability Volume 67, Dec. 2016, pp. 64-73

F. Firouzi, S. Kiamehr and M. B. Tahoori, “NBTI mitigation by optimized NOP assignment and insertion,” 2012 Design, Automation & Test in Europe Conference & Exhibition (DATE), Dresden, 2012, pp. 218-223.

Haider Muhi Abbas, Mark Zwolinski, and Basel Halak. Aging Mitigation Techniques for Microprocessors Using Anti-aging Software. Chapter 3, Ageing of Integrated Circuits - Causes, Effects and Mitigation Techniques, Springer, Cham. ISBN 978-3-030-23781-3

Synopsys Digital Standard Cell Lib. SAED_EDK32/28_Core Databook.

A. Calimera, M. Loghi, E. Macii and M. Poncino, “Dynamic Indexing: Leakage-Aging Co-Optimization for Caches,” in IEEE Transactions on Computer-Aided Design of Integrated Circuits and Systems, vol. 33, no. 2, pp. 251-264, Feb. 2014

Freddy Gabbay is an associate professor and the head of the Computer Science Department at the Ruppin Academic Center, Israel. He received his Ph.D. in electrical engineering from the Technion – Israel Institute of Technology, Haifa, Israel. His research interests include microelectronics, computer architecture, and machine learning. Prof. Gabbay is a member of the IEEE and IEEE Computer Society.

Avi Mendelson is a visiting professor in the Computer Science and Electrical Engineering Departments at the Technion – Israel Institute of Technology, Haifa, Israel. He received his Ph.D. in electrical engineering from the University of Massachusetts. His research interests include computer architecture, security, and real-time systems. Prof. Mendelson is an IEEE Fellow.

Basel Salameh is a verification engineer at Apple. He received his B.Sc. in electrical engineering from the Technion – Israel Institute of Technology, Haifa, Israel. His research interests include computer architecture, security, and machine learning.

Majd Ganaiem is a physical design CAD engineer at Apple, studying for a B.Sc. in computer engineering at the Technion – Israel Institution of Technology, Haifa, Israel. His research interests include computer architecture, networks, and machine learning.

www.opencores.org