Article

Deep Neural Network Memory Performance Modeling and Simulation Framework

Citation: Gabbay, F.; Lev Aharoni, R.; Schweitzer, O. Deep Neural Network Memory Performance Modeling and Simulation Framework. Mathematics 2022, 9, x. https://doi.org/10.3390/xxxxx Academic Editor: Oliviu Matei Received: 30 September 2021 Accepted: Not accepted yet. Published: date Publisher’s Note: MDPI stays neutral with regard to jurisdictional claims in published maps and institutional affiliations. Copyright: © 2021 by the authors. Submitted for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license (https://creativecommons.org/licenses/by/4.0/).

Freddy Gabbay 1,*, Rotem Lev Aharoni2 and Ori Schweitzer2

1	Engineering Faculty, Ruppin Academic Center, Emek Hefer 4025000, Israel

2	Electrical and Computer Engineering Faculty, Technion – Israel Institute of Technology;

rotaharoni@gmail.com, ori.sch@campus.technion.ac.il

*	Correspondence: freddyg@ruppin.ac.il

Abstract: Deep neural networks (DNNs) are widely used in various artificial intelligence applications and platforms, such as sensors in IoT devices, speech, and image recognition in mobile systems, and web searching in data centers. While DNNs achieve remarkable prediction accuracy, they introduce major computational and memory bandwidth challenges due to the increasing model complexity and the growing amount of data used for training and inference. These challenges introduce major difficulties not only due to the constraints of system cost, performance, and energy consumption but also due to limitations in currently available memory bandwidth. The recent advances in semiconductor technologies have further intensified the gap between computational hardware performance and memory systems bandwidth. Consequently, memory systems are today a major performance bottleneck for DNN applications. In this paper, we present DRAMA, a Deep neuRAl network Memory Simulator. DRAMA extends the SCALE-Sim simulator for DNN inference on systolic arrays with a detailed, accurate, and extensive modeling and simulation environment of the memory system. DRAMA can simulate in detail the hierarchical main memory components – such as memory channels, modules, ranks, and banks – and related timing parameters. In addition, DRAMA can explore tradeoffs for memory system performance and identify bottlenecks for different DNNs and memory architectures. We demonstrate DRAMA’s capabilities through a set of experimental simulations based on several use cases.

Keywords: machine learning; deep neural networks; systolic array; memory hierarchy; DRAM memory; performance simulation.

1. Introduction

The usage of deep neural networks (DNNs) is continuously growing in various applications, such as IoT, edge devices, mobile phones, and high-performance servers in cloud platforms and data centers [1–3]. In recent years, DNNs have demonstrated phenomenal prediction and classification capabilities in various applications, including image recognition [4], natural language processing [5], and recommendation systems [6]. However, the remarkable accuracy of DNNs comes with a high computational complexity and huge memory system throughput for both network training and inference. These challenges have become major entry barriers for DNNs on various computational platforms due to both the platforms’ limits in performance, cost, and energy, and their limited memory system bandwidth.

In recent years, there has been significant growth in the use of dedicated accelerators [7, 8], such as systolic arrays (SAs) and graphics processing units (GPUs), which can offer more efficient processing of DNNs than general purpose CPUs. An SA is a spatial computing system that consists of a network of processing elements that rhythmically perform computations and pass data through the system. A GPU is a single instruction multiple data (SIMD) architecture. Similar to SAs, GPUs consist of arrays of processing elements that run the same operation simultaneously; however, GPUs have two fundamental differences from SAs:

- While GPU processing elements are programmable and execute the same operation simultaneously, SA processing elements can only execute a fixed operation (usually a multiply-and-accumulate).

- Unlike the SA, however, the processing elements in GPUs cannot pass data directly but rather must exchange data via memory. This introduces a major benefit for the SA since it utilizes memory bandwidth more efficiently.

Both SAs and GPUs are highly dependent on the performance and throughput of their memory systems. This has been further intensified in recent years with advances in semiconductor technologies, which has led to an exponentially growing gap between the speeds of compute elements and memory systems [9].

In this paper, we introduce DRAMA, a Deep neuRAl network Memory Simulator. DRAMA extends the SCALE-Sim [10] simulator, which is commonly used to mimic DNN inference on SAs, with a detailed, accurate and extensive simulation environment of the associated memory system. SCALE-Sim is a cycle-accurate functional simulator that can process matrix multiplications and convolutions for various DNNs architectures. SCALE-Sim allows designers and architects to perform architectural and performance exploration by simulating various DNNs with different SA configurations while considering various system tradeoffs. While SCALE-Sim can accurately model memory accesses performed by the SA to the local memory, it lacks accurate modeling of the main memory system. Due to the growing impact of memory systems on DNN processing performance, it is essential to model the complete memory system accurately. To overcome this issue, DRAMA offers an extension to SCALE-Sim that can simulate the detailed building blocks of main memory, such as memory channels, modules, ranks and banks, and related timing parameters. In addition, DRAMA offers a broad range of configuration parameters that can help in exploring various memory system configuration tradeoffs and identifying bottlenecks for different DNN architectures. This is further demonstrated in Section 4 where we present an architectural exploration for various ResNet-18 [11] memory systems using the DRAMA simulation environment.

We summarize the contributions of this paper as follows:

- We introduce DRAMA, a novel open-source simulation environment that can perform detailed, accurate, and extensive simulation of SA memory systems.

- DRAMA can run either as an extension to the SCALE-Sim simulator to model the entire memory system accurately or as a standalone main memory simulator.

- DRAMA offers a broad range of configuration parameters, which can help to perform an accurate memory system exploration for DNNs and to understand the interplay between key memory system parameters.

- DRAMA generates an accurate main memory trace file that takes into account main memory system configuration and related timing parameters.

- We demonstrate the impact of memory system parameters on SA performance and memory system throughput through an experimental analysis of several case studies.

The remainder of this paper is organized as follows: Section 2 provides related background and reviews previous work. Section 3 describes the DRAMA simulation environment architecture and flow of operation. Section 4 presents the experimental results. Finally, Section 5 summarizes our conclusions.

2. Background and Prior Works

We start our background discussion by providing an overview on systolic arrays and their memory system. Next, we provide an overview of prior work, and in particular the SCALE-Sim environment that is extended by this study.

2.1. Systolic Arrays

Systolic arrays (SAs) belong to the group of spatial architectures that are commonly used for DNN convolution or general matrix multiplication [1, 12]. An SA (illustrated by Figure 1) consists of a two-dimensional mesh array of tightly coupled processing elements (PEs) that can forward data directly between PEs in the same row and column through unidirectional connections. Each PE has an arithmetic logic unit (ALU) – which is typically a multiply-and-accumulate functional unit – and a local storage element that is used to store intermediate computations. Matrix multiplication performed by the SA multiplies the input feature map matrix (INFMAP) by the filter weights and produces the output feature map (OFMAP). The INFMAP and filter weights are read from the SA local memory, and the OFMAP is written to the local memory. One of the key advantages of SAs is their mesh structure, which can save a significant number of memory accesses by taking advantage of reusing forwarded data that have been processed by neighboring PEs.

Figure 1. Schematic illustration of a systolic array (SA).

2.2. Memory System

The SA memory system is depicted in Figure 2 and includes three key elements: a local memory, single or multiple memory controllers, and a main memory. The local memory, which is located near the SA, is typically a small and fast memory. The SA continuously accesses the local memory through the process of matrix multiplication. The local memory stores three double buffers (also termed ping-pong buffers) for the IFMAP, OFMAP, and filter weights [1, 12]. The double buffers in the local memory allow the SA to execute the matrix multiplication in parallel with fetching the next IFMAP and weights from main memory and writing the previous OFMAP to main memory. All main memory access requests are enqueued onto the memory controller job queue, which manages all read and write access to the main memory [13]. The SA can employ multiple memory controllers, where every controller is connected to a different memory channel. Multiple memory channels can perform parallel memory access operations and thereby increase memory bandwidth. Every memory channel consists of one or more dual in memory modules (DIMMs). A DIMM consists of multiple memory chips that are soldered to the DIMM. There are two DIMM configurations: a single rank DIMM and a dual rank DIMM. In a single rank DIMM, the memories are soldered onto one side of the DIMM, while in a dual rank DIMM the memories are soldered onto both sides. Using a dual rank configuration increases the DIMM storage capacity by a factor of two.

Figure 2. Memory system scheme of a systolic array.

The memory chips in DIMMs are dynamic random-access memories (DRAMs) [14], which are slower than the local memory but have a significantly larger storage capacity. As illustrated in Figure 3, every DRAM chip is organized into multiple memory banks. A bank consists of a two-dimensional matrix of memory elements, and each element is uniquely associated with an individual address. Memory elements within the same row have consecutive memory addresses. The memory address provided by the SA for read or write operations is partitioned into a row-select part and a column-select part. The row-select activates an access to a single row (also known as a page), while the column-select accesses the particular memory element within the row. Consecutive memory accesses within the same row in a bank can be performed quickly since the row has been already activated, and the individual memory elements can be directly accessed from the previously read row. This mode of operation is termed fast page mode (FPM). When a new memory access switches to a different row, the time required for the access will become longer since the new row needs to be activated.

Figure 3. Schematic of a memory bank.

One of the methods of enhancing main memory efficiency is to increase the number of memory banks, since larger numbers of banks will increase the likelihood of FPM access. The number of banks and pages that are available for FPM access in the main memory is given by Equation 1.

| Total number of banks = CDRB | (1) |
| --- | --- |

where C is the number of memory channels, D is the number of DIMMs, R is the number of ranks and B is the number of banks.

The efficiency of the memory system is crucial to SA performance and throughput. It is necessary for the next IFMAP and weights to be ready in the local memory before the current matrix multiplication is completed. In addition, the previous OFMAP must also be written to main memory prior to the completion of the SA. If the memory system is a bottleneck, the SA will become stalled.

2.3. SCALE-Sim

SCALE-Sim [10] is a cycle-accurate functional simulator that mimics SA accelerators for DNNs. It can simulate both matrix multiplication and the convolutions that are used by many DNN models. SCALE-Sim allows designers to configure different SA architectures and validate their performance and throughput while considering various tradeoffs. It can also model the memory system in a limited level of a detail. The memory accesses run by SCALE-Sim are limited to the local memory, where it simulates the double buffers of IFMAPs, OFMAPs, and weights. SCALE-Sim generates cycle-accurate read accesses to the local memory, which are required for continuous SA operation. In addition, it accurately models the write accesses that are the outputs of the SA to the OFMAP buffer in the local memory. It should be noted that SCALE-Sim models the main memory using a single configuration parameter that represents the main memory average bandwidth. It does not model the main memory channels, DIMMs, ranks, banks, or the detailed timing parameters related to the main memory. In addition, SCALE-Sim generates trace files, which consist of all memory accesses for IFMAPs, OFMAPs, and weights and can be used for offline processing and analysis of memory traffic. To guarantee continuous operation of the SA, SCALE-Sim also uses the main memory traces to determine whether the prefetching of the data required for the SA operation (INFMAP and weights) and the writing of the previous OFMAP have been completed prior to the next computation task.

2.4. Prior Related Simulation Environments

Memory system simulation environments can be partitioned into three classes: statistical, cycle-accurate, and RTL-based. Statistical simulators typically use a statistical model for the memory system latency [15–17]. Such simulators offer the advantage of a high simulation speed; however, they incur a lack of accuracy, which can vary significantly between different use cases. Cycle-accurate simulators accurately model the memory system, but at the cost of longer execution time. In recent years, several cycle-accurate DRAM simulators have been developed for general applications [18–21]. These simulators are tightly coupled to the evolving DRAM JDEC standard, and some of them are also tightly coupled to general purpose single-core and multi-core CPU simulators [22–24]. RTL-based simulators [25] use hardware description models (HDLs) to accurately simulate DRAM controllers and memory system elements; however, they suffer from a lack of scalability and require long simulation times.

3. DRAMA Simulation Environment Framework

In this study, we present DRAMA, a deep neural network memory hierarchy simulator. DRAMA offers a plug-and-play extension to the SCALE-Sim simulator and can either be simply integrated with other DNN simulators or run as a standalone simulator. DRAMA offers a highly configurable simulation environment, which can be very useful for system architects to explore different memory hierarchies, topologies, and related parameters and tune overall SA and memory system performance. DRAMA is designed in python and is provided as open-source code in GitHub. We start by introducing DRAMA operation flow, describe its architecture, and discuss the implementation details. Next, we present DRAMA’s configuration settings, and finally, we describe the details of the simulator outputs.

3.1. DRAMA Architecture

Figures 4 and 5 illustrate DRAMA’s operation flow and block diagram, respectively, in conjunction with SCALE-Sim. The block diagram and flow represent the software implementation of DRAMA (the hardware representation of DRAMA is illustrated in Figure 6). SCALE-Sim simulation consists of three main functions:

- A simulation of the SA processing and generation of local memory access traces.

- A simulation of the requests targeted to the local memory ping-pong buffers.

- Generation of the trace to the main memory for prefetching the next INFMAPs and filter weights and for writing the previous OFMAPs.

As illustrated in Figure 5, SCALE-Sim reads the topology parameters of the neural network from the topology file [10]. The file specifies the number of layers and type, INFAMP dimensions, and filter dimensions. The configuration file, which will be described later, is used to specify the simulation parameters for both SCALE-Sim and DRAMA.

Figure 4. SCALE-Sim and DRAMA operation flow.

As part of DRAMA’s software architecture, we have designed a software interface such that all main memory access traces from SCALE-Sim are channeled into DRAMA. This software interface allows co-simulation of DRAMA and SCALE-Sim to take place simultaneously. DRAMA reads the main memory traces and generates main memory requests, which are forwarded to the detailed main memory simulation engines. The requests, which are initiated by different sources (INFMAPs, filter weights, and OFMAPs), are arbitrated by the request arbiter and are enqueued to one of the main memory channel queues. The mapping is done such that every memory region (defined by start address and end address) is mapped to an individual channel. Then, all accesses in the queues are arbitrated by the channel arbiter. For every request that is scheduled for main memory access, DRAMA performs the following actions:

- It calculates the overall time required by the DDR channel to serve the request.

- It updates the simulator performance statistics counters.

- It removes the request from the channel queue.

Once the request processing is done, the main memory access details are written to the trace file. This process is repeated for as long as there are access requests in the channel queue. Once the requests in all queues have been processed, the simulator checks whether additional traces have been generated. If there are new main memory access traces, the process will be repeated, as illustrated in Figure 4; otherwise, the simulation will be terminated, and DRAMA will generate the simulation statistics. DRAMA simulation outputs are described in detail in Section 3.3.

Figure 5. DRAMA block diagram.

The main memory system hardware architecture that is simulated by DRAMA is illustrated in Figure 6. The system consists of a set of DRAM controllers, where each controller is assigned to an individual DRAM channel. Each DRAM controller compromises:

- An arbiter, which arbitrates the stream of memory requests destined for the DRAM channel. The arbiter operates in a round-robin (RR) manner and adds the requests to the request queue.

- A request queue, which includes all pending requests. The queue is managed in a first-in-first-out (FIFO) manner.

- A scheduler, which schedules the request in the head of queue to be served by the DRAM channel as soon as it becomes available. The scheduler removes the served request from the memory controller queue.

The DRAM channel topology can be viewed as a tree structure. Every channel consists of multiple DIMMs of single or dual rank. All memory chips in a rank operate as a single logical memory, which is partitioned into multiple banks. The number of channels, DIMMs, ranks, and banks are specified in the configuration described in Section 3.2.

Figure 6. DRAMA system architecture scheme.

3.2. DRAMA Configuration Settings

The configuration file is parsed by the DRAMA simulator and is used to specify the detailed architecture of the main memory system. The configuration file, which is summarized in Table 1, specifies the number of DRAM channels and size, the number of DIMMs per channel, the DIMM ranks, the number of banks, and the page size. In addition, it defines the memory channel data bus width and the DRAM timing parameters. The configuration also specifies the method of mapping memory addresses to DRAM channels. The mapping can be done using either the least significant or most significant bits of the address. In the former, consecutive memory addresses are mapped to different memory channels. In the latter, the entire memory space is partitioned into multiple windows where each is associated with an individual memory channel.

Table 1. DRAMA configuration file parameters.

| Configuration Parameters |  |
| --- | --- |
| NumerOfChannels | The number of DRAM channels. |
| ChannelMapping | Defines the mapping of memory address to a DRAM channel. The address mapping can be based on the least significant bits or the most significant bits of the memory address. |
| NumerOfDIMMs | The number of DIMMs per memory channel. |
| NumberOfRanks | The number of DIMM ranks. |
| NumberOfBanks | The number of banks. |
| BusSize | Memory channel data bus width. |
| PageSize | DRAM page size. |
| ChannelMemorySize | The memory size of a channel. |
| AddressMapping | The DRAM address mapping mode: row interleaving or cache block interleaving. |
| CacheBlockSize | The cache block size when AddressMapping is set to cache interleaving. |
| OpenPageAccessTime | The access time (in clock cycles) to an open page (FPM). |
| ClosedPageAccessTime | The access time (in clock cycles) to a closed page. |

Finally, the configuration file specifies row, column, and bank mappings within a channel using two common methods – row interleaving and cache line interleaving – which are illustrated in Figure 7. In row interleaving, consecutive accesses are mapped to consecutive banks, while cache line interleaving maps consecutive cache blocks to consecutive memory banks [26].

Figure 7. Row, column, and bank mapping modes.

3.3. DRAMA Outputs

The simulation performed by DRAMA produces two output files: a statistics summary file and a trace file. The statistics summary file, illustrated in Figure 8, summarizes for every convolution or matrix multiplication task the following performance-related statistics of every channel in the memory system:

- Channel memory bandwidth, measured in bytes per cycle.

- Average number of clock cycles per memory request.

- The percentage of time the SA was idle.

- The total number of pages open in a DRAM channel.

- Busy cycles – the number of cycles when the memory channel was busy.

- Total number of requests.

- Total number of bytes read by the SA.

Figure 8. DRAMA statistics summary output.

The trace file, illustrated in Figure 9, is a csv file where every line lists the addresses of the main memory prefetch accesses and their accurate occurrence time. DRAMA’s trace file extends the traces generated by SCALE-Sim by performing a detailed simulation of the main memory system. In particular, the timing annotation in every line takes into account the overall memory system configuration and related timing parameters.

Figure 9. DRAMA output memory trace file.

4. Experimental Results and Discussion

Our experimental analysis demonstrates the capabilities of the DRAMA simulation environment for various computational workloads and use cases. The presented experiments are mainly focused on the impact and contribution of the memory system and its impact on overall SA performance.

4.1. Experimental Use Case

The use case for our experimental study is the ResNet-18 convolutional neural network [11]. ResNet-18 is 18 layers deep and is commonly used for image classification tasks. ResNet-18 can classify images into 1000 object categories. As part of the experimental use case, we have chosen four different convolution layers from ResNet-18, which are presented in Table 2. Each of the chosen layers has multiple repetitions in the network, so they fairly represent the overall network workload.

Table 2. DRAMA configuration file setting parameters.

| Configuration parameters | IFMAP dimensions | Filter dimensions | Number of channels | Number of filters | Stride |
| --- | --- | --- | --- | --- | --- |
| Conv1 | 56x56 | 3x3 | 64 | 64 | 1 |
| Conv2 | 28x28 | 3x3 | 128 | 128 | 1 |
| Conv3 | 14x14 | 3x3 | 256 | 256 | 1 |
| Conv4 | 7x7 | 3x3 | 512 | 512 | 1 |

Our experimental exploration examines two memory system parameters – the DRAM bandwidth and the number of memory channels – on overall SA performance and memory system throughput. The results are presented and discussed in Section 4.2.

4.2. Case Studies Analysis

In the first use case, we examine the impact of DRAM bandwidth on a single channel memory system with eight DIMMs and eight banks in every DIMM. The channel mapping is based on row interleaving, and the page size is 4096 bytes. The experiments examine DRAM bandwidths of 2, 8, 16, and 32 bytes per SA clock cycle. The SA dimension used in our simulations is 16x16, which runs output stationary dataflow. Figure 10 illustrates simulation results for the four convolution layers in Table 2. The experimental results measure the percentage of time the SA was stalled, the percentage of time the memory system was idle, the SA relative performance (compared to an ideal SA with an ideal memory system), and the memory system's overall throughput. It can be seen that as the DRAM bandwidth increases, the percentage of time the SA is stalled decreases. In addition, we observe that different convolution layers introduce different memory throughput requirements. For example, for conv1 a DRAM bandwidth of 8 bytes per clock cycle is sufficient to eliminate the SA from stalling, while conv2 requires 32 bytes per cycle to avoid SA stalls. Figures 10a–d also show that the SA relative performance is inversely correlated with the SA stalls: once the number of SA stall clock cycles drops to 0, the SA relative performance reaches 100%. Figures 10a–d also indicate that as we increase the DRAM bandwidth, the overall memory system throughput utilized by the SA grows. This, however, may increase the percentage of time the memory system is idle. In certain cases, we observe that although the memory system is idle for a certain amount of time, there are still stalls in the SA. This is because the DRAM access takes longer whenever a new page is accessed, and as a result, the DRAM is idle till the new data is accessed and the SA is stalled.

(a)

(b)

(c)

(d)

Figure 10. The impact of DRAM bandwidth on SA performance and memory system throughput: (a) conv1 layer, (b) conv2 layer, (c) conv3 layer, and (d) conv4 layer.

In the second use case, we examine the impact of the number of channels and the channel mapping on the SA performance and memory system throughput. We run the simulations with one, two, and four memory channels, where the two- and four-channel cases have each been run using the two different channel mapping options – using either the least significant bits (LSB) of the memory address or the most significant bits (MSB). The memory system configuration assumes a DRAM bandwidth of 16 bytes per SA clock cycle and a total of 4 DIMMs with 8 banks per DIMM. The page size is 2048 bytes, and the address mapping is row interleaving.

Figure 11 summarizes the simulation results for all the convolution layers in Table 2. It can be seen that when increasing the number of channels, the overall SA performance is improved and the number of SA stalls is reduced. In addition, it can be observed that LSB channel mapping achieves better performance than MSB channel mapping for all convolution layers. This can be explained by the fact that when using LSB channel mapping, consecutive memory addresses are mapped to different memory channels, while in MSB channel mapping they are mapped to the same channel. Since the SA generates consecutive memory accesses, the mapping to different channels allows serving the accesses in parallel and thereby improves overall memory throughput and SA performance. From the results presented in Figures 11a–d we can also conclude that using four channels with LSB mapping achieves the best performance in all convolution layers, since it enables the SA to reach the maximum possible performance. The most noticeable speedup in the SA performance is achieved in the conv1 and conv2 layers, where the four channels with LSB mapping scenario gains over 40% improvement in SA performance relative to a single memory channel. When the number of channels increases, this not only helps improve the SA performance, but also increases the overall memory system performance. The total number of bytes prefetched from the system remains the same regardless of the number of channels; however, as the number of channels increases, the overall memory access time gets smaller, and thereby the throughput is improved significantly.

(a)

(b)

(c)

(d)

Figure 11. The impact of the number of channels on SA performance and memory system throughput: (a) conv1 layer, (b) conv2 layer, (c) conv3 layer, and (d) conv4 layer.

5. Conclusions

DNN processing introduces not only major computational complexity but also continuously growing memory throughput requirements. Memory systems have a crucial impact on DNN processing performance, and thus it is essential to accurately model them when exploring various DNN architectures and system configurations in simulation environments. In this paper, we introduce DRAMA, an open-source simulation environment for deep neural network memory systems. DRAMA extends the SCALE-Sim simulator with detailed, accurate, and extensive simulation of the memory system. DRAMA accurately models DRAM channels, DIMMs, ranks, banks, and related timing parameters. In addition, it provides a comprehensive range of configuration parameters, which can help in examining memory system architectural tradeoffs. DRAMA can run either as an extension to SCALE-Sim or as a standalone simulator. We have demonstrated DRAMA’s capabilities on four representative convolution layers from ResNet-18, where we examined the effects of DRAM bandwidth, number of channels, and bank mapping on SA performance and memory system throughput.

Author Contributions: Conceptualization, F.G.; methodology, F.G.; software, O.S. and R.L.-A.; validation, F.G., O.S. and R.L.-A.; formal analysis, F.G., O.S. and R.L.-A.; investigation, F.G.; resources, F.G.; data curation, F.G.; writing – original draft preparation, F.G.; writing – review and editing, F.G.; visualization, F.G. O.S. and R.L.-A. All authors have read and agreed to the published version of the manuscript.

Funding: This research received no external funding.

Data Availability Statement: The DRAMA simulator code is available as open source available at: https://github.com/DRAMA-technion/DRAMA (September 10, 2022).

Conflicts of Interest: The authors declare no conflict of interest.

References

- Sze, V., Chen, Y., Yang, T., & Emer, J.S. Efficient Processing of Deep Neural Networks: A Tutorial and Survey. In: Proceedings of the IEEE, 105, 2295–2329. 2017.

LeCun, Y., Bengio, Y., & Hinton, G. “Deep learning,” Nature, vol. 521, no. 7553, pp. 436–444, May 2015.

Esteva, A., et al. “Dermatologist-level classification of skin cancer with deep neural networks,” Nature, vol. 542, no. 7639, pp. 115–118, 2017.

Krizhevsky, A., Sutskever, I., & Hinton, G.E. “ImageNet classification with deep convolutional neural networks,” In: Proc. NIPS, 2012, pp. 1097–1105.

Collobert, R., Weston, J., Bottou, L., Karlen, M., Kavukcuoglu, K., & Kuksa, P. “Natural language processing (almost) from scratch,” J. Mach. Learn. Res., vol. 12,
pp. 2493–2537, Nov. 2011.

Covington, P., Adams, J., & Sargin, E. Deep Neural Networks for YouTube Recommendations. In: Proceedings of the 10th ACM Conference on Recommender Systems 2016 (RecSys '16). Association for Computing Machinery, New York, NY, USA, 191–198. https://doi.org/10.1145/2959100.2959190.

- Jouppi, N.P., et al. “In-datacenter performance analysis of a tensor processing unit,” CoRR, vol. abs/1704.04760, 2017. [Online]. Available: http://arxiv.org/abs/1704.04760

Morgan, T.P. “Nvidia pushes deep learning inference with new pascal GPUs,” Next Platform, Tech. Rep., Sep. 2016.

- Carvalho, C. "The gap between processor and memory speeds." In: Proceedings of IEEE International Conference on Control and Automation. 2002.

- Samajdar, A., Zhu, Y., Whatmough. P., Mattina. M., & Krishna T. SCALE-Sim: Systolic CNN accelerator simulator. arXiv preprint arXiv:1811.02883. 2018 Oct 16.

- He, K., Zhang, X., Ren, S., & Sun, J. Deep. Residual Learning for Image Recognition. In: Proceedings of the 2016 IEEE Conference on Computer Vision and Pattern Recognition (CVPR), Las Vegas, NV, USA, 27–30 June 2016.

Jacob, B. The Memory System: You Can’T Avoid It, You Can’T Ignore It. Morgan and Claypool Publishers, You Can’T Fake It (2009)

Carter, B., Hsieh, W.C., Stoller, L., Swanson, M.R., Zhang, L., et al. Impulse: Building a smarter memory controller. In: Proceedings of the International Symposium on High-Performance Computer Architecture, 1999.

Double Data Rate (DDR) SDRAM Standard. JEDEC JESD79F Standard, February 2008.

Li, S. & Jacob, B. Statistical DRAM modeling. In: Proceedings of the International Symposium on Memory Systems, MEMSYS ’19, p. 521-530. Association for Computing Machinery, New York, NY, USA (2019). https://doi.org/10.1145/3357526.3357576

Jung, M., Feldmann, J., Kraft, K., Steiner, L., & Wehn, N. Fast and accurate DRAM simulation: Can we further accelerate it? In: IEEE Conference on Design. Automation and Test in Europe (DATE), pp. 364–369. Grenoble, France (2020)

Todorov, V., Mueller-Gritschneder, D., Reinig, H., & Schlichtmann, U. Automated construction of a cycle-approximate transaction level model of a memory controller. In: Proceedings of the Conference on Design, Automation and Test in Europe, DATE ’12, pp. 1066–1071. EDA Consortium, San Jose, CA, USA (2012). http://dl.acm.org/citation.cfm?id=2492708.2492972

Rosenfeld, P., Cooper-Balis, E., & Jacob, B. “DRAMSim2: a cycle accurate memory system simulator.” IEEE Comput. Archit. Lett. 10(1), 16–19 (2011). https://doi.org/10.1109/L-CA.2011.4

Li, S., Yang, Z., Reddy, D., Srivastava, A., & Jacob, B. “DRAMsim3: a cycle-accurate, thermal-capable DRAM simulator.” IEEE Comput. Archit. Lett. pp. 1–1 (2020). https://doi.org/10.1109/LCA.2020. 2973991

Kim, Y., Yang, W., & Mutlu, O. “Ramulator: A fast and extensible DRAM simulator.” IEEE Comput. Archit. Lett. PP(99), 1–1 (2015). https://doi.org/10.1109/LCA.2015.2414456

Jeong, M.K., Yoon, D.H., & Erez, M. DrSim: a platform for flexible DRAM system research. http:// lph.ece.utexas.edu/public/DrSim. Accessed 15 Aug 2019

Binkert, N., et al. “The gem5 simulator.” SIGARCH Comput. Archit. News 39(2), 1–7 (2011). https:// doi.org/10.1145/2024716.2024718

Rodrigues, A.F., et al. “The structural simulation toolkit.” SIGMETRICS Perform. Eval. Rev. 38(4), 37–42 (2011). https://doi.org/10.1145/1964218.1964225

Sanchez, D., & Kozyrakis, C. “ZSim: fast and accurate microarchitectural simulation of thousand-core systems.” SIGARCH Comput. Archit. News 41, 475 (2013). https://doi.org/10.1145/2508148. 2485963

- Sudarshan, C., Lappas, J., Weis, C., Mathew, D.M., Jung, M., & Wehn, N. A Lean, Low power, low latency DRAM memory controller for transprecision computing. In: Pnevmatikatos, D.N., Pelcat, M., Jung, M. (eds.) Embedded Computer Systems: Architectures, Modeling, and Simulation, pp. 429–441. Springer, Cham (2019)

- Zhang, Z., Zhu, Z. & Zhang, X. A permutation-based page interleaving scheme to reduce row-buffer conflicts and exploit data locality. In: Proceedings 33rd Annual IEEE/ACM International Symposium on Microarchitecture. MICRO-33 2000, 2000, pp. 32–41, doi: 10.1109/MICRO.2000.898056.

https://github.com/DRAMA-technion/DRAMA