Jan. 13, 2023

HISTORY OF FREE ELECTRON LASERS IN ISRAEL AND ESTABLISHMENT OF CENTER FOR COMPACT ACCELERATOR DEVELOPMENT IN ARIEL

(English synopsis)

AVRAHAM GOVER

In memoriam of:

Yigal Cohen Orgad and Professor Yuval Neeman – Leaders of the FEL project

Haim Kleinman, Jerzy Sokolowski, Arie Eichenbaum, Miki Kanter, Boris Kapilevitch, and Dimitri Borodin –Project pioneers

This monograph presents the history of free electron laser (FEL) research and development in Israel and focuses on the establishment of the Israeli FEL center, which is a joint project between Tel Aviv University, Ariel University, and the center for compact accelerator development in Ariel.

1. Free Electron Laser

Fig 1: Schematic illustration showing the structure of a free electron laser (FEL). An electron beam is accelerated to high energy in an electron accelerator and injected into a periodic magnetic structure (called the “wiggler”). Due to transverse oscillation of the electrons, electromagnetic radiation (i.e., light) is emitted. The radiation frequency increases with increasing electron energy (from microwaves to x rays). The illustration shows an FEL oscillator (laser oscillator) in which the radiation is trapped and amplified between two mirrors. In most applications, the laser is built in the configuration of an amplifier (FEL amplifier): a light beam is injected together with the electron beam, is amplified, and exits after the wiggler.

Fig 2: Conceptual illustration of the FEL-based laser weapon system of the Strategic Defense Initiative based on a ground platform. The system is designed to intercept missiles with a nuclear warhead in flight in outer space before re-entering the atmosphere. The powerful laser beam produced at the output of the FEL is transmitted over a kilometer so that it expands, thereby avoiding damage to the relay mirror, which directs the beam out of the atmosphere. "Fighting mirrors" in space aim and focus the beam on the target.

Fig 3: The world's first X-Ray FEL (Standford Linear Accelerator Linac Coherent Light Source) lased for the first time on April 10, 2009 in the wavelength range 1.5–15 angstroms. The FEL is based on a kilometer-long linear accelerator (Linac) producing energies of 13.6 GV and a wiggler (undulator) 112 m long.

2. The early start of FEL research in Israel

3. The Tandem-FEL project at the Weizmann Institute

Fig 4: Experimental free electron maser (FEM) system at Tel Aviv University. The photograph shows laboratory engineer Haim Kleinman dismantling the system in preparation for its transfer to Ariel. Today, the instrument is in the visitor center at the entrance to the accelerator center in Ariel and is used to demonstrate the operating principles of the Superradiant FEL.

Fig 5: First FEL facility at the Weizmann Institute was built by the FEL consortium after converting an electrostatic ion accelerator (TANDEM van der Graaff) to an electron accelerator. The first millimeter wave ring was demonstrated on Hanukkah, 1997.

4. Moving the Tandem-FEL project to Ariel

Fig 6: Crane removing the tandem accelerator from the Weizmann Institute by "Avi Manofim."

Fig 7: Yigal Cohen-Orgad (center) with the accelerator in the background after its transfer to Ariel. On his right are Gover and Sokolovsky, on his left is the team of Romanian accelerator experts who packaged the accelerator.

Fig 8: Tandem accelerator in its new residence in the accelerator building in Ariel. The aluminum tube at the end of the accelerator transfers the FEL radiation into the user room through a 1.5-m-thick protective wall. Standing next to the accelerator is Mickey Kanter, accelerator engineer.

Fig 9: Visit of Minister of Science, Raleb Majadla, to FEL center in 2009. In the photo are Professor Dan Meirstein, president of the college, Yigal Cohen-Orgad, Professor Avraham Gover, head of the Ariel Accelerator Center, and minister Rafi Eitan.

Fig 10: Professor Gover receiving the 2005 FEL Award at the International FEL Conference at Stanford, USA for his contribution to the development of the Superradiant FEL concept (which later formed the basis for the construction of the THz Superradiant FEL at the Ariel Accelerator Center). Standing to the right of him are John Galayda, chairman of the conference and, to the left, Alexander Van der Meer, chairman of the award committee

http://www-ssrl.slac.stanford.edu/lcls/fel2005/index.php

5. The THz Superradiant FEL based on an RF-LINAC accelerator

6. Status update of the Accelerators center and the FEL project 2022

Fig 11: Optically-pumped RF-gun accelerator device (ORGAD).

Fig 12: Accelerator for radiation chemistry applications.

Fig 13: A three-floor construction project at the Ariel Accelerator Center. Funding provided by the Schlesinger family: Construction started in 2022.

Fig 14: The planned building for the Ariel Accelerator Center and the visitor lobby. Completion scheduled for 2024.

Fig 15: A didactic model by Mickey Kanter in the visitors' lobby to explain the principles of operation for the FEL and the Superradiant FEL. The model is based on the free electron maser (FEM) experimental system of Tel Aviv University (see Figure 4), which was transferred to the Ariel Accelerator Center.

Fig 16: The Israeli Super radiant THz FEL facility, October 2022. In the back is the ORGAD accelerator, in the front is the magnetic wiggler and the radiation out-coupling. Standing from left to right are Ariel Naus, Avraham Gover, Aharon Friedman, and Yehiel Vashdi.

Fig 17: THz radiation source and the radiation transfer line from the accelerator hall to user rooms.

Fig 18: First measurement of superradiative THz radiation on Oct 31, 2022.

References

- https://en.wikipedia.org/wiki/Strategic_Defense_Initiative

- https://en.wikipedia.org/wiki/Free-electron_laser

- https://www.ynet.co.il/news/article/h1uh0lsv9

- https://lcls.slac.stanford.edu/

- A. Gover and A. Yariv, Collective and single electron interactions of electron beams with electromagnetic waves and free electron lasers, Appl. Phys. 16, 121–138 (1978).

- I. Shraga, C. Leibovitch, S. Eckhouse, Y. Goren, and A. Gover, Doppler shifted cyclotron maser radiation pumped by an asymmetric undulator, Appl. Phys. Lett. 46, 936–938 (1985).

- M. Arbel, A. Abramovich, A. L. Eichenbaum, A. Gover, H. Kleinman, Y. Pinhasi, and I. M. Yakover, Super-radiant and stimulated super-radiant emission in a pre-bunched beam free electron maser, Phys. Rev. Lett. 86, 2561–2564 (2001).

- A. Gover, Superradiant and stimulated superradiant emission in prebunched electron-beam radiators – part I: Formulation, Phys. Rev. Spec. Top. 8, 030701 (2005).

- https://breakingdefense.com/2021/03/israel-seeks-us-help-on-lasers/

- A. Abramovich, M. Canter, A. Gover, J. S. Sokolowski, Y. M. Yakover, Y. Pinhasi, I. Schnitzer, and J. Shiloh, High spectral coherence in long pulse and continuous free electron laser measurements and theoretical limitations”, Phys. Rev. Lett. 82, 5257 (1999).

- יוסי גולדשטיין, :ללא מעצור – סיפור חייו של יגאל כהן-אורגד" הוצאת אוניברסיטה אריאל בשומרון.

- Y. Socol, A. Gover, A. Eliran, M. Volshonok, Y. Pinhasi, B. Kapilevich, A. Yahalom, Y. Lurie, M. Kanter, M. Einat, and B. Litvak, Coherence limits and chirp control in long pulse free electron laser oscillator. Phys. Rev. Spec. Top. 8, 080701 (2005).

- A. Gover, R. Ianconescu, A. Friedman, C. Emma, N. Sudar, P. Musumeci, and C. Pellegrini, (2019). Superradiant and stimulated-superradiant emission of bunched electron beams. Rev. Mod. Phys. 91, 035003 (2019).