Post-Contact Behavior of an Axially Compressed

Fiber inside a Flexible Cylinder –

Experimental and Numerical Investigations

# Yossi Dayan†, David Durban**, Sefi Givli†*

# †Faculty of Mechanical Engineering, **Faculty of Aerospace Engineering, Technion – Israel Institute of Technology, Haifa 32000, Israel

# *Corresponding author, email: givli@technion.ac.il

# February 15, 2021

# Abstract

This paper analyzes the post-buckling behavior of a clamped-clamped elastic fiber constrained inside a flexible cylinder theoretically, numerically and experimentally. It focuses on characterizing the contact configuration between the fiber and the flexible cylinder wall. The elastic fiber is variously compressed up to its elasticity limit where a segment of the fiber makes contact with the cylinder wall and its deformation becomes three-dimensional (3D). The analysis was performed using various tools, including representative experiments, image processing for the experimental results, finite element (FE) analysis of the experimental system, and analytical models. To this end, we employ a novel experimental method, with a setup consisting with f a flexible cylinder, which, to the best of our knowledge, represents the first experimental work where experimental compression on a flexible fiber within a cylinder with flexible walls can be observed. This is combined with image processing and synchronized force measurements. Additionally, the movement within the flexible cylinder’s centerline was measured as a function of the force placed upon the fiber using image processing. The results are consistent with published theoretical predictions that are based on a simplified theoretical model assuming a perfect fiber. This study provides insights into the behavior of such systems and may have practical implications in the fields of stent procedures, medical endoscopy, deep drilling, and the mechanics governing the growth of roots and plants, which characterize the flexible fiber’s movement within a flexible medium. This differs from research work carried out using a rigid cylinder that does not best simulate behaviors in the aforementioned systems.

Keywords — Fiber, post-buckling, contact, flexible, cylinder

1  Introduction

The post-buckling behavior of an elastic fiber subjected to lateral constraints is of practical importance in a number of fields where access to enclosed spaces with an elastic fiber is needed, ranging from medical procedures (such as in vivo diagnosis) to engineering applications. In the field of oil production, for example, the drilling is carried out using a very long cylinder that serves to drill holes [1–4]. When the drill is subjected to axial load, it may buckle and touch the wall of the borehole. Applications in the sphere of medical procedures include inserting fibers for the purpose of medical imaging or cardiac catheterization, as well as inserting a long catheter into a patient's blood vessel for the deployment of stents for treating artery disease [5–8]. When the leading end of the catheter encounters a narrowing or a lesion, the catheter may bend too much and touch the arterial wall. In invasive microsurgery, flexible catheters are manipulated in order to reach organs requiring treatment [9–10]. In these applications, the elastic fiber's behavior can affect the interactions between the elastic fiber and the environment. The simplest mechanical model for oil drilling is a thin winding fiber contained within a rigid cylinder with a circular cross-section. Research of on this issue regarding a rigid cylinder began as early as the 1950s and has been widely explored [1–3]. In contrast, in applications involving an artery and human tissue, such as the development of filopodia in living cells [11–14], it is not possible to assume that the cylinder is rigid. However, according to a review of articles about a fiber contained within an elastic cylinder, it appears that no relevant experimental research exists about this question. This current research aims to address this gap by substituting an elastic-walled cylinder for rigid-walled cylinders in experimental studies [15]. As described in previous works examining cases where the fiber is helical, it is possible to use an energy system to predict the relationship between the applied load and the size of the helix [16–19]. In inclined [20–25] and horizontal [26–31] cylinders, gravity acts on the fiber on the lower side of the cylinder to stabilize it. This stabilization allows the fiber to withstand a significant load. In curved cylinders [32–34], it was found that the buckling load that causes the sinusoid shape and helix of the fiber is usually much greater than those in straight cylinders. Using geometrical calculations, the theory of elasticity [35–40] was developed, among other things, to investigate the post-buckling behavior of the fiber along the cylinder wall. The end effects of the post-buckling behavior of a contained curved fiber that is subject to an axial load are discussed [41], see Figure 1. This work refers only to the final stage of deformation, which is a point-line-point connection, and it has been reported in the literature [42–45] that before reaching a point-line-point deformation, the fiber passes through different deformation patterns. While all previous studies have assumed a rigid cylinder wall, in reality, a totally rigid cylinder does not exist. As previously noted, in certain applications, especially biological and medical ones, an elastic cylinder wall would be a better model. This work aims to replace the assumption of a rigid cylinder as explored in previous studies [45] with that of an elastic one. There are [12, 46–50] studies that address the behavior of a fiber within an elastic-walled constraint, but the subject has not yet been empirically examined thoroughly with a three-dimensional and numerical investigation of the fiber's deformation within an elastic cylinder. The intention here is to test the cylinder's sensitivity to failure after loads are applied to the wall by means of the fiber, to confirm the theoretical results empirically and numerically, and to find an analytical model for evaluating the problem.

GuidewireLesioL

Guidewire

Artery channel

Force

Figure 1: (a) A guidewire passes through the lesion area of an artery channel. It is assumed that the guidewire is squeezed by the elastic tissue in the lesion [6]; (b) The experimental setup, with a flexible cylinder with a diameter.

# 1.1 Description of the problem

In cases where the fiber is threaded into the cylinder and begins to experience two-dimensional deformation up to its contact with the cylinder wall, it is assumed that the wall exerts a radial force. The intensity is relative to the radial movement of the wall. We assume that the cylinder wall behaves similarly to the base of the spring as it appears in the study [51], with the spring constant β. The spring model as it appears in study [51] is a very basic analogy to a fiber on an elastic wall, but it helps simplify our problem. The spring constant is not a value that is usually measured. However, we assume that there is a good proportional relationship between the spring constant and the Young’s modulus of the material of which the elastic wall is composed. As a close approximation, the Young’s modulus of elasticity of the material will be chosen as our spring constant. Similarly, in the case of oil drilling, we assume that the outer and inner radii of the drill casing are 0.125 m and 0.04 m respectively. The length of the drill casing between the two stabilizers is 50 m [52]. Assuming that the drill casing and the borehole walls are made of steel, with a Young's modulus of 200 GPa, β is approximately 1010. If the wall of the borehole is made of shale, with a Young's modulus of 20 GPa [53], then β is approximately 109. In these cases, it is likely that the elasticity of the cylinder wall is not taken into consideration.

Shifting attention to a stent deployment application with heart disease patients, among the areas with the highest risk of formation of a fatty layer in the blood vessels are along the main left artery and the frontal left descending artery [54], with a total length of approximately 45 mm [55]. The Young's modulus of the artery is approximately 1.2 MPa [56,57]. The radius of the fiber inserted into the stent is 0.18 mm, with a Young's modulus of 200 GPa. Given these rough estimates, the dimensionless spring constant β is on the order of 104. In this case, it is probably necessary to take the elasticity of the arterial wall into account. This research represents the first time that a large number of experiments have been conducted about the problem of the behavior of fibers in a cylinder. In these experiments, a thin fiber is subject to load, is exposed to the wall of an elastic cylinder [58], which continues to exert load on it, as occurs in the case of the treatment of blood vessels and other tissues in the human body.

# 1.2 Goal, Motivation and Achievement

# The goal of this study is to test the behavior of a fiber threaded into a flexible cylinder using axial force. The motivation for the study is to conduct a broad empirical, numerical investigation to find solutions for preventing damage to a fiber found in the wall of a flexible cylinder, such as in the cases of blood vessels, tissues, and similar engineering applications, including deep drilling. This study

could have significant implications for applications through its comprehensive empirical investigation of the behavior of the fiber and the flexible cylinder, its finite element simulations, its comparisons with the existing literature, and its adaptation to a dimensionless basic analytical model that may best help achieve the objective of this work. In the case of a fiber within a flexible cylinder, we expect to see two types of deformation of the external cylinder. The first, Type 1, or "Local" deformation, causes changes in the shape of the cross-section in the area in which the fiber makes contact with the cylinder. The second, Type 2, or "Global" deformation, causes a global change in the shape of the cylinder; specifically the straight axis of the cylinder becomes curved. To the best of our knowledge, this research project is original, as there is no known publication or project that has undertaken an empirical investigation of the combined behavior of a flexible fiber within a flexible cylinder. The exception of [46] only involved numerical research, based on assumptions that significantly simplified the subject; i.e., it involved only local deformation with no coupling, and only Type 1 deformation was involved. Specifically, the assumption in that study was that the cylinder was actually a two-dimensional and not a three-dimensional deformation. In essence, when the fiber makes contact with the cylinder, the cylinder exerts reactive force in the area of contact that is proportional to the extent of "penetration" of the fiber beyond the surface of the cylinder. However, this does not reflect reality, as the cylinder is not treated as a real elastic structure. That is, there is no effect or coupling between the radial sliding only at point A and at point B, even if it is very close to it. However, this is a good limit case in that it enables significant simplification of the analysis, focusing only on Type 1 (local) deformation. Another important point is that [46] carries out only a numerical analysis (no finite elements, but rather the solution of a set of non-linear differential equations). In addition, [46] has no experimental element. In view of the great complexity of the general problem of a fiber within a flexible cylinder, we propose executing an additional "small step" that would enable analysis of a problem that is somewhat less complex than the most general case, but still allows for insights. Based on these insights, we can then continue on to more general and complex cases. Therefore, for our study, we have selected a "reverse" limit case chosen from [46]. We examine the case in which only Type 2 deformation occurs, i.e., global deformation of the cylinder, with any local deformation negligible. This actually is the case when the circumferential/tangential stiffness of the cylinder is very large in relation to the lengthwise stiffness. This means that these cylinders do not allow a local change of the shape or size of the cross-section, but do enable global deformations of the cylinder, where the cylinder changes from straight to curved. For the experiments, we use non-standard cylinders, which are actually commonly used for cladding or protection of another cylinder inserted into them. The flexible cylinders used are made of polymeric material, but with a thin rigid material fiber passing helically in their circumference, see Figure 1(a). In terms of simulations of finite elements, to create a model of the cylinder, we use a composite material that is very rigid in the circumferential direction. Another benefit of approaching the problem in this way, where there are only global deformations, is that it makes it highly likely that a relatively simple mathematical model can be created, which may even allow for analytical insights, at least regarding the initial deformation. Specifically, the flexible cylinder can be modeled as a beam and the fiber within it makes contact at its center and exerts force on it. Another practical advantage of performing experiments with the flexible cylinders as described is that they can be obtained without initial curvature, unlike other flexible cylinders that usually have a uniform initial curvature (rolled up): see Figure 1(b). Another important point is that the numerical study in this case is not for the purpose of confirming the experiments. Rather, it is rather an inseparable part of the investigation: the process involves performing experiments. From these experiments, we can then derive graphs of force versus shortening expressing the shortening of the axial length of the fiber while its original length is maintained, as well as obtain information regarding the response of the flexible cylinder. In contrast to experiments with a rigid cylinder, we cannot gather information regarding the behavior of the fiber inside the cylinder, since the rigid cylinder was transparent and, in our case, it is opaque. To understand the behavior of the fiber, we use finite element simulations. In the first stage, we want to ensure  that the responses we obtain correspond to the experimental measurements with respect to the force-shortening graph and the response of the external cylinder. Once such correspondence is found that enables us to conclude that the simulations indeed model the behavior very well, we can complete the picture of the experiment. Using simulations we can recreate the behavior of the fiber within the cylinder, including where there is contact and what type of contact: e.g., point, linear, planar or three-dimensional deformation, etc.

Our work is expected to be innovative in several ways. It represents the first systematic empirical investigation within a flexible cylinder, the first time global deformation of the cylinder is examined, and the first time that a combination of experiments, finite element simulations, and a simple analytical model used. This approach enables us to construct a complete picture of the behavior of a fiber inside a flexible cylinder, the response of the flexible cylinder, and the interaction between them. The results of these experiments are compared to numerical research of similar cases in an effort to find an analytical model that describes the process of failure of the wall of the elastic cylinder following the continued application of load by the fiber after its contact with the wall. Drawing on the results of this research project we will be able to better understand the characteristics of the failure and how it can be prevented.

# 2 Materials and methods

# Description of system

The theoretical predictions assume the following: the thin elastic fiber of length with circular cross-section is inextensible and unshearable. The fiber is uniform in mechanical properties along its length  and is stress-free when it is straight and untwisted, its deformation is constrained inside a flexible cylinder with a diameter D and the centerline of the constraining flexible cylinder coincides with the unstressed straight fiber. The diameter, d, of the fiber cross-section is negligible compared to that of the flexible cylinder. We consider the deformation of the fiber when it is subject to prescribed edge-thrust and under the constraint of the cylinder. It is assumed that the fiber is completely fixed at one end and not able to rotate around the longitudinal axis. At the other end, the fiber is clamped laterally but is free to slide longitudinally. According to small-deformation theory, the 1-point contact deformation exists only in planar form; while in the elastica model, the 1-point contact deformation of the spatial form also exists [46]. As expected, the difference between the small-deformation theory and the elastica model grows as the radius of the constraining cylinder becomes larger.

# 2.2 Experimental system

Experiments were performed with an Instron 4483 machine, on which the designated experimental system was installed: see Figure 2(a). The experimental system includes a CSN EN 10270-1 steel wire fiber with a length of L=500 mm and a fiber diameter of d=1.8 and 2.4 mm inside various flexible cylinders with diameters of D=32, 42 and 74 mm and a rigid cylinder with a diameter of D=42 mm. Flexible cylinders come in large rolls from the manufacturing plant. Therefore, curvatures exist in cylinders of short lengths, as can be seen in Figure 2(b). During the experiment, a small torque was applied to their ends in an attempt to align them without stretching them.

During the first stage, tensile and squeeze experiments were carried out on the flexible cylinders without fibers to examine their characteristics. The flexible cylinders without fiber, were bound at their ends to impose clamped boundary conditions at both ends and then stretched and squeezed using an Instron. In order to carry out the first stage of the experiments, a dedicated array was set up, as described in Figure 2(a), with adapters adjusted to hold only the flexible cylinder, which could then be stretched and squeezed without compressing the fiber. Another preliminary experiment, designed to determine the spring constant of the cylinder and several experiments to measure the force exerted on the centers of the three cylinders, D=32, 42 and 74 mm, were carried out by stretching the centers of the flexible cylinders without fibers. To measure the applied force and the center’s movement, a screw was affixed precisely in the center of the cylinder, to which a steel cable was connected. The other end of the cable was connected to a strain gage. These steps were taken when the cylinder was in a horizontal position and fastened at both ends. This set-up enabled the applied force to the center and the movement of the cylinder’s center to be measured. In Figure 3 is a description of the experimental array using the same set-up as that employed for all the fiber compression experiments, only without the fiber within the flexible cylinder.

For the main experiments where the fiber is compressed, which constitute the bulk of the study, special adapters were designed and installed to impose clamped boundary conditions at both ends of the fiber. Then, the lower adapter was fixed to the flexible cylinder while the upper one was attached to the moving arm of the Instron machine, so the fiber coincided with the symmetrical axis of the cylinder at the start of the experiment, as described in Figure 4.

During the experiment, the distance between the two ends of the fiber was slowly decreased in a quasi-static movement. Using the Instron machine the shortening rate of the ends was 10 mm/s. This process resulted in the bending deformation of the fiber constrained by the flexible or rigid cylinders. Our method in which the distance between the two ends of the fiber is shortened while the length of the fiber remains constant differs from the method in [28], where the fiber is injected from left to right and pulled over two feeder rollers through a slave injector, forming a slack loop before being pulled through a primary injector into the constraining glass cylinder. The advantages of this experimental method over those used in previous studies is that involves minimal friction and greater precision in measuring the fiber force. In our experiments, reaction forces are transmitted over an air-bearing slider to the force sensor. The fiber is then pulled through a channel by an idler wheel and a drive wheel driven by a servo-stepper motor located close to an acrylic clamp holding the pipe in place. The deformation is examined for fiber a diameter of d=1.8 and 2.4 mm and for other inner diameter values of the flexible cylinders: d=32, 42 and 74 mm. Here, L=500 mm is the free length of the fiber in the initial unloaded state, i.e., the distance between the two clamping points at the beginning of the experiment. An end state shortening of the fiber is determined by the displacement of the upper clamp controlled by the Instron machine using the displacement control method. In this configuration, loads are applied to a part based on the displacement, and the displacement is determined using an Encoder installed on the Instron. In this method, the displacement changes incrementally while the reaction force depends on the stiffness of the structure. Axial compressive force applied on the fiber was measured by a static load cell. Together with displacement, both were synchronized with a digital camera, Type MAKO G-223 with a CMOSIS/ams CMV2000 sensor, global shutter and 50 frames per second that was used to record the experiment. The maximum level of shortening was restricted by software to prevent fiber plastic deformations.

In each experiment, two complementing characteristics of the response were recorded: the force displacement relationship and details of cylinder movement. In order to determine these features, the axial force was applied to the fiber along with the corresponding shortening of the ends. Analysis of the force-displacement relationship provides the core information on the fiber loading process, revealing important aspects of the fiber behavior. The details of the effect on the flexible cylinders caused by the fiber deformation were determined by analyzing the successive frames taken by the camera and complemented with MATLAB® assisted image processing, that allows for clear representation of the distance between the centerline of the flexible cylinder and the initial centerline of the constraining flexible cylinder. Synchronization between the camera and the Instron machine facilitates the configuration of the contact to be identified and related directly to the force-displacement relation. With this synchronization, qualitative and quantitative comparison between the behavior observed in the experiment and the structural response predicted by FE simulations and by the analytical model can be achieved.

Figure 2: (a) The tensile and compression experimental set up, with flexible cylinders with diameters of . (b) Flexible cylinders with 32, 42 and 74 mm diameters are pictured in order to assess the curvature of the cylinder.

Figure 3: The force-movement relationship experimental set up, with flexible cylinders of diameters .

bc

a

b

Fiber

Flexible

Cylinder

Instron

Base

Figure 4: (a) Schematic description of the main experiment and system components. (b) The experimental setup, with flexible cylinders with diameters of .

# 2.3 Finite-element simulations

FE simulations were performed with the commercial FE software ABAQUS FEA. A static analysis was designed to simulate the experimental system, which included a 500 mm fiber clamped at both ends and laterally constrained by flexible and rigid cylinders. A static stress analysis is used when inertia effects can be neglected, but takes into account rate-dependent plasticity and hysteretic behavior for hyperelastic materials. The model of the fiber and the cylinder are meshed with hexahedral solid elements, type C3D8R (8-node brick, accounting for geometrical nonlinearity), with over 50 elements in the fiber cross-section and a total of 1600 elements in the fiber and 2000 elements in the cylinder. Tube elements’ size was reduced in areas of contact between the fiber and the cylinder: see Figure 5. A Young’s modulus of  was assigned to the fiber, in accordance with tensile experiments performed with the Instron machine and orthotropic characteristics for flexible with hyperelastic constant C10=2.715,C01=-2.348 and polycarbonate with Young’s modulus of 2540 MPa for a rigid cylinder. Preliminary analyses with high-order brick elements and with a larger number of elements in the mesh resulted in similar results. Boundary conditions were implemented by defining zero-displacement of all degrees of freedom associated with the nodes at the two ends of the fiber, with the exception of the vertical displacement of the upper end, which was gradually increased during the simulation. As shown in Figure 5, the fiber at the end of one side (where no force is applied) is fixed to the x, y, and z axes for both displacement and rotation around each axis. At the other end of the fiber, where the force is applied, the fiber is fixed to rotate on the three axes and cannot rotate around them. On the two other axes that are not parallel to the movement of the end of the fiber, the fiber’s end is fixed and cannot move in the direction of these axes. On the axis that is parallel to the movement of the end of the fiber, the fiber’s end has a constraint that enables it to move in parallel to the axis for a defined displacement of 15mm, consistent with the motion in the experiment.

During a static step, a time period is assigned to the analysis. The “time” increments are then simply fractions of the total period of the steps. In these cases, a quasi-static solution can be obtained only if the magnitude of the load does not follow a prescribed history; it must be part of the solution. The time incrementation may be controlled in a quasi-static analysis directly and here, it is controlled automatically by ABAQUS.

In the numerical analysis, the vertical displacement represents the shortening between the two ends of the fiber, as described in Section ‎2.1. The vertical force on the upper end of the fiber, which is the force applied by the Instron machine in the experiment, was determined in the simulation. The shortening rate of the ends was 10 mm/s, which is comparable to the rate at which the experiments were performed. Preliminary FE simulations showed that lower rates produced similar results, indicating that our system behaves as a quasi-static system. In the numerical analysis, a contact between the cylinder and the fiber was defined using penalty stiffness in the normal direction of the contact surfaces, pressure-overclosure with "hard" contact, and no penetration.

Figure 5: Mesh model at ABAQUS software towards making a HEX element analysis

2.3.1 ABAQUS Simulations Flexible Wall Cylinder results

Figure 6 Presents the results of a simulation for a flexible cylinder with a diameter of D=42 mm and fibers with diameters of d=1.8 and 2.4 mm. The graph describes the force that develops at the end of the fiber on which axial force is applied as a function of its shortening. In order to examine whether hysteresis exists in the ideal state, force on the fiber was released. The results of these released applications can also be seen in Figure 7. Here, the simulation helps not only identify and explain what forces are acting upon the flexible cylinder wall, but also analyze the nature of the contact. This process is critical, as the cylinder is opaque, and, as a result, information can be obtained only through a simulation. The letters (a)–(h) refer to developing deformation stages in the cylinder and the fiber, as can be seen in Figure 7. Figure 8 displays the force acting on the flexible cylinder wall from the fiber deformation versus end shortening, as obtained in the simulation. It is: 4.6 N for a 1.8 mm diameter fiber and 6.5 N for a 2.4 mm diameter fiber.

a

b

Figure 6: Vertical force versus end shortening — FE simulation results for flexible cylinders D = 42 mm, L = 500 mm: (a) d=1.8 mm. (b) d=2.4 mm.

Figure 7: Results of FE simulations showing the deformation of the fiber and contact with the flexible cylinder wall, for: d=1.8 , D = 42 mm, L = 500 mm, R=55 mm. Top view (all images are at an identical scale).

a

b

Figure 8: Wall force versus end shortening - FE simulation results for flexible cylinders D = 42 mm, L = 500 mm: (a) d=1.8 mm. (b) d=2.4 mm.

2.3.2 ABAQUS Simulations Rigid Wall Cylinder

In order to carry out a comparison of the behavior of a fiber and a flexible cylinder, a simulation was performed on a rigid cylinder with a diameter of D = 42 mm together with a fiber with a diameter of d=1.8. Figure 9. describes the force that develops at the end of the fiber on which pressure is applied as a function of its shortening. The letters (a)–(h) refer to developing deformation stages of the fiber. The letters (a)–(h) refer to developing deformation stages in the cylinder and the fiber, as visually displayed in Figure 10. The maximum force on the flexible cylinder wall from the fiber deformation, as obtained in the simulation, is 8.7 N, which is greater than that for the flexible cylinder.

Figure 9: Vertical force versus end shortening — FE simulations results for rigid cylinder:d=1.8 mm, D = 42 mm, L = 500 mm.

Figure 10: Results of FE simulations showing the deformation of the fiber and contact with the rigid cylinder wall, for: d=1.8 mm, D = 42 mm, L = 500 mm, R = 55 mm. Top view (all images are at identical scale)

Figure 11 illustrates a numerical simulation of the presence of different behavior for a fiber having a diameter of d=1.8 mm. This is similar to what happens in the experiment when the fiber is within a flexible or stiff cylinder, both at the first contact stage and, subsequently, with greater shortening values.

Figure 11: Vertical force versus end shortening — FE simulation results for flexible and rigid cylinders: L=500 mm, and fiber: d=1.8 mm.

# Analytical insights from initial imperfection analysis

In this section, we present analytical derivations for three key features associated with the behavior of the fiber. The first and second features are the end displacement (shortening) of the fiber at the onset of first contact between the fiber and the cylinder with symmetric and antisymmetric imperfections as illustrated in Figure10, and the third is the load at which the transition from 2D (planar) to 3D deformation occurs. The analysis assumes a linear stress-strain relation (Hooke’s law).

- End displacement for first contact

The analysis in this section is based on a well-established elastic solution of a free fiber [17] (see Figure 12).

Figure 12: Description of elastica problem for a clamped-free fiber

There,

where

(2)

and are the complete elliptic integrals.

We denote  as the lateral displacement at the middle of the fiber, as illustrated in the Figure 13.

Figure 13: Description of boundary conditions and post-buckling response of the fiber in this research

In order to obtain the lengths of segments L1andL2, let sθ represent the arc length given by

(3)

where  is the vector defining the location on the fiber and  is the angle of the tangent at this point with respect to a constant direction, e.g. .

Boundary condition (6)–(8) is used in solving the clamped-free problem by inserting the following relations into the solution of the clamped-free problem:

2L1+L2=L

(4)

(5)

(6)

The parameters are unknown; however, the following conditions must exist:

  - Identical load in both segments:

(7)

  - Identical bending in the intersection of two segments:

(8)

Equations (3) (combined with (6)–(8)) can be viewed as providing the relationship between  through a single parameter. In other words, for a given value of , each member of the triad

can be easily calculated. This approach can be used to drawas a function of.

(9)

In addition, a similar approach can be used to plot the elastic solution for the relation between  and:

This relation is a geometrical property of the curve that describes the fiber center-line, and it does not depend on the cross-section geometry or on the material of the fiber. The agreement between the exact solution and the approximation is evident. Thus, (11) can safely be used to calculate the value of  where the fiber makes the first contact with the rigid cylinder.

Figure 14 shows the computational results for the mathematical model of the fiber's first contact with the cylinder wall. The curves describe the shortening of the fiber as a function of the cylinder’s radius; that is, the shortening required for a first contact with the cylinder wall to occur for cylinder of various radii. There are two curves. The first curve was made according to a full computation, including employing elliptic integrals. The second curve represents an approximate computation without using these integrals.

Figure 14: End shortening versus radial displacement — compared analytical model results,

approximation and without approximation model calculation.

- Cylinder deformation analysis

      - 2.4.2.1 Small Cylinder deformation analysis

In this case there are two different expressions for the bending moment corresponding to the two portions of the beam .

Figure 15: A clamped beam subjected to a concentrated load

Eq. for the deflection curve must therefore be written for each portion of the cylinder. In this way we obtain

Since the two branches of the deflection curve have a common deflection at the point of application of the load, the two expressions  must be identical for . From this it follows that . Finally, we need to determine only two constants C and C2, for which determination we have two conditions, namely, that the deflection at each of the two ends of the beam is zero. Substituting x=0 and y=0 in the first of expressions, we find:

.

Substituting  in the second of expressions ,we obtain

.

Substituting the values 13 and 14 of the constant into the Eq. solution for the deflection curve, we obtain:

The first of these equations gives the deflections for the left portion of the beam and the second gives the deflections for the right portion.

Substituting the value (14) into Eq. (12) solution, we obtain

.

The slope at any point of the deflection curve can readily be calculated from these equations, although we often need the values of the slope at the ends of the beam.

Substituting  in the first part of Eq. , in the second, and denoting the slopes at the corresponding ends by , we obtain

.

The maximum deflection occurs at the point where the tangent to the deflection curve is horizontal. If , as in Figure 15, the maximum deflection is evidently in the left portion of the beam. We can find the position of this point by equating the first of the Eq. to zero to obtain l2-b2-3x2=0, from which

.

This is the distance from the left support to the point of maximum deflection. To find the maximum deflection itself, we substitute Eq. (18) in Eq. (15) , which gives

.

If the load  is applied at the middle of the span the maximum deflection is evidently also at the middle. Its magnitude is obtained by substituting  in Eq. (10), which results in

.

From Eq. (18) it can be concluded that in the case of one concentrated force, the maximum deflection is always near the middle of the beam. When b=l2 it is in the middle.

      - 2.4.2.1 Large cylinder deformation analysis

Figure 16 shows a beam with uniform EI and cross-sectional area , fixed at both ends and subject to a vertical load at the centerure. The deflected curve is longer than the straight line between  and . This elongation develops axial forces N in the beam in addition to shear forces and bending moments.

y

In order to remain within the elastic range,  cannot exceed a certain value, which in turn also limits the deflection . Therefore, the problem will be analyzed applying the moderately large deflection theory of beams by using the approximate expression for the curvature.

Figure 16: A clamped beam subjected to a concentrated load

For reasons of symmetry only the right-hand half of the beam in Figure 16 will be analyzed. The bending moment at a point x,w is:

where  is the fixing moment (clockwise) at the point , see Figure 17.

is axial displacement.

In addition:

.

The axial force  is calculated from Eq. with Eq.

.

Note that  is constant but unknown. In order to make use of the kinematic boundary conditions, let us integrate both sides of Eq. with respect to

.

The beam is clamped at both ends, so using zero boundary conditions for , the axial force is related to the square of the slope by

.

We define non-dimensional forces and moment as:

.

Now the normalized equilibrium equation in the vertical direction becomes

Where

Now, using Eq. and Eq., we write the non-dimension axial force:

where for the case of a thin-walled circular cross-section  is equal to the (non-dimensional) radius of the tube. The solution of Eq.  is

where . The first two terms are the solution of the homogeneous equation and the last two terms correspond to a particular solution. The unknowns  and  are found by inserting the particular solution into Eq.  and comparing coefficients, while the remaining unknowns C1,C2 and M0 are determined from the boundary condition.

N

N

N//

y

y

N

Figure 17: The right-hand half of beam for reasons of symmetry

Accordingly, we have

and

.

Also, using the last boundary condition, , we write

.

The equation of the elastic line can be found by substituting  into Eq.

.

At  the deflection is , so

.

Taking the first derivative of Eq.  and squaring it, Eq. provides a constant axial force N as

and, since  , we rewrite Eq.as

.

Combining this expression for  with Eq.

.

Although it is not possible to write an explicit relation between  and , Eqs.  and  provide an implicit relation through . This nonlinear force-deflection relationship is illustrated in Figure 18. and is a good match. It is instructive to compare this result to the linear force-displacement relationship predicted by the small-deformation analysis,  when  is derived from Eq. . This can be seen in Figure 19. It is evident that the accuracy of the small-deformation analysis is insufficient once deflections are larger than . For example, a deflection of  corresponds to . Next, we look for an explicit approximate relation between  and . To this end, we write a Taylor expansion of Eq.  near the linear-case () and neglect terms of order  and higher:

.

Next, using  and , gives

P=λ3λ-2tanh(λ2)-1δ 				 .			(40).									(4

Finally, by inserting Eq.  into Eq.  we arrive at the explicit (approximate) force-displacement relationship

.

Figure 18 shows that the accuracy of this approximate relation is very high. In terms of dimensional quantities, relation  is written as

where  and .

Figure 18: Normalized force vs. normalized deflection

Figure 19: Normalized force/linear force vs. normalized deflection

- Fiber small-deformation analysis

In this section, we analyze the initial (planar) deformation stages of a fiber that is constrained by a flexible cylinder. For this, we conceptually replace the cylinder with an equivalent spring that applies the same force to the fiber (see Figure 20). For simplicity, and in accordance with the experimental results, we assume that the deflection of the fiber is small, y≪L0, such that the behavior of the fiber is governed by the standard linear equations of bending. We denote this with L0 as the initial fiber length, as the fiber diameter, and R1 as the inner radius of the flexible cylinder and the actual (current) distance between the location of the spring and the original location of the fiber. Hence, the original gap between the fiber and the constraining spring is R0, as shown in Figure 20. The fiber is clamped at one end x=0 and supported by a horizontally movable slider at the other x=L0. The fiber is subjected to an external compressive force P acting along its long axis that results in a horizontal displacement, , of one end. After buckling, the fiber deforms and makes contact with the cylinder/ spring at the midspan x=H (see Figure 20). Further increases in the external force (or alternatively, of ) results in a larger deflection, which is governed by the resisting force of the spring. We note that a similar analysis was performed in [48–49], where a slender beam constrained by a sprung wall with linear behavior was studied theoretically and experimentally. Here, however, the force-displacement relationship of the constraining spring is cubic in Eq. . Thus, we follow the analysis of [48] and extend it to account for the cubic behavior of the spring.

Figure 20: A clamped fiber is subjected to an axial compressive forceand is subsequently constrained by the wall of a flexible cylinder, here replaced conceptually with a spring.

Treating the fiber as a thin line, , its vertical displacement at the contact point becomes

.

In order to enable non-dimensional analysis, all lengths are normalized by  and all forces by . Accordingly, the deflection of the fiber is governed by the (non-dimensional) equation:

where boundary conditions are dependent on contact conditions, as discussed below. In addition, the horizontal displacement,, is determined by:

.

Symmetry allows for consideration of only half of the fiber, between x=0x=0 and x=1.

2.4.3.1 Analysis of Pre-buckling and buckling of a clamped-clamped fiber

Three stages of the fiber behavior are considered: before contact, during buckling of the fiber, and during the fiber-cylinder contact stage. The solution of the buckling problem for a clamped-clamped segment with no contact is

well established and is presented here for completeness. During the first stage, the fiber experiences a horizontal displacement, but does not flex and remains horizontal:

During the second stage, the force  reaches a critical buckling level, . The buckling of the fiber occurs instantaneously and the fiber “jumps” to contact the non-deformed cylinder equivalent at the point y(1)=R0, where R0 was defined earlier (see Figure 20). The horizontal displacement  becomes:

.

During the third stage, displacement of the cylinder equivalent occurs due to contact with the fiber. A linear analysis of this process is presented in the next section.

2.4.3.2 Point contact

Once the fiber makes contact with the spring, the resistance of the spring to additional deflection needs to be taken into account. Thus, the boundary conditions are

Here,  is the spring force , i.e.:

Solving Eq. with boundary conditions (48), using Eq.leads to the relationship .

From Eq. , the solution for  takes the form:

.

By substituting Eq. (48) into Eq. (50), all unknown coefficients are defined:

The equation of the elastic line Eq. is expressed as:

Note that , the force in the spring, is unknown. It can be a characteristic of the spring, namely :

.

Spring force  in Eq.for a linear spring, when  is:

.

Therefore, the equation of the elastic line for linear spring with Eq. (53) is:

Using the definition of the displacement from Eq. (45) and substituting Eq.(54) for a linear spring force for from Eq. (49) when (α3=0), the normalized horizontal displacement becomes

.

Now we calculate the spring force PS in Eq.(49) for a non-linear spring, when α3≠0, defined as

Multiply Eq.(49) in the β(k) and rearrange the equation

.

From Eq. (56), the spring force PS for a non-linear spring, is

..

Fig 21 displays the deflection of the fiber  Vs. ,when  for linear spring force from Eq. (54) and  for non-linear spring force from Eq.(51) with Eq. (57).

Figure 21: The deflection of the fiber  Vs.  ( ) — compared results.

Fig. 22(a) displays the behavior predicted by small-deformation theory for the symmetrical case with a linear spring with different α1, P Vs. Δ, with  from Eq. (55). Figure 22(b) displays the behavior predicted by small-deformation theory for the symmetrical case with linear and non-linear spring force compared to experiment, PPcr Vs. .  for non-linear spring force from Eq.(45) with  for non-linear spring force and Eq. .

Figure 22: The behavior predicted by small-deformation theory for the symmetrical case. Measured vertical force versus end shortening for a flexible-walled cylinder: L=500 mm, D= 42 mm and fiber:. (a) linear spring force (). (b) linear and non-linear spring force and experiment result — comparison graph.

As can be seen in the Figure 22 (b), comparing the linear spring force to the nonlinear spring force and to the experiment, there are small differences in the onset of force activation which are reset at larger P, y,PS and Δ for non-linear spring force for Figure 22 (b) were calculated separately.

# Results

3.1 Preliminary experiment

3.1.1 Tensile and Squeezing Flexible Cylinder

a

b

During the first stage of this research, tensile and squeezing experiments were carried out on the cylinders without fibers to examine the characteristics of the cylinder material. Figure 23 presents the results of stretching experiments on the flexible cylinders. Figure 24 presents the results of compression experiments on the flexible cylinders.

c

Figure 23: Vertical force versus cylinder end lengthening for flexible cylinder in tensile experiment, (a) . (b). (c).

b

a

c

Figure 24: Vertical force versus end cylinder shortening for flexible cylinder in squeeze experiment, (a) . (b). (c).

Figure 25: Vertical force versus end shortening for flexible cylinders, D=32, 42 and 74 mm in squeeze experiment — compared results.

It is noticeable in Figure 25, a cylinder with a diameter of D=74mm, requires greater force in order to obtain similar shortening. Flexible cylinders with diameters of D=32 and 42 mm exhibit similar behavior.

3.1.2 Examination of the flexible cylinder spring constant

To determine the spring constant of the cylinder, continuous sampling was taken of the applied force on the cylinder center and the movement of the flexible cylinder from its starting centerline. For some of the flexible cylinder diameters, experiments were carried out until there was a movement of 55 mm of the cylinder’s center from the central axis at the start of the experiment: see Figure 26. Each experiment was carried out in three iterations to test for hysteresis; that is, until reaching the required lengthening of the flexible cylinder and then releasing to the initial zero shortening position.

Figure 26: Vertical force versus axial distance for examination of the flexible cylinder spring constant, (a) D=32 mm . (b). (c).

Comparison for examination of the flexible cylinder spring constant:

3.2. Shortening Fiber Experiments

3.2.1 Examining the critical loading on a fiber before it contacts the cylinder

Figure 27: Vertical force versus axial distance for examination of the flexible cylinder spring constant, cylinders — compared results.

The summary of results in the comparative graphs of the experiments, Figure 26, indicates that with small movements of the flexible cylinder’s center 10mm, the relationship between the movement and the force is linear; with larger movements in cylinders with D=42 and74 mm, the relationship is polynomial.

During the second stage of the research, experiments were carried out with fibers. Compression (shortening) experiments were conducted with fibers having diameters of D=1.8 and 2.4 mm, and with three different cylinder diameters of D=32, 42 and 74 mm as can be seen in the experiment results in Figures. 28 and Figure 29. In these experiments, the fibers were shortened by up to 15 mm, with the specific shortening determined by examining the critical load on a fiber before contact with the cylinder wall. Each experiment was carried out three times to test for hysteresis; that is, until reaching the required fiber shortening and then releasing it to the starting point with zero shortening. In all the experiments with the fibers, the reaction force at the end of the fiber resulting from its shortening was measured, with the distance between the two ends of the cylinder remaining unchanged. The initial contact with the cylinder wall on its return to the starting state is the result of the 74 mm diameter cylinder's visco-elastic behavior, causing disengagement between the fiber and the tubing with a descending value differing from that of the ascending one. Xcn.= First Contact, Pcr = Critical Buckling (Euler) and X3D = Three-Dimensions Deformation.

b

a

c

Figure 28: Measured vertical force versus end shortening for d=1.8mm and three different flexible cylinders: L = 500 mm (a) D = 32mm. (b) D = 42mm. (c) D = 74mm.

b

a

c

Figure 29: Measured vertical force versus end shortening for  and three different flexible cylinder:  (a) . (b) . (c) .

3.2.2 Analysis of fiber and flexible cylinder behavior through the fiber compression experiment

In the fourth stage of the research, an analysis of the cylinder's behavior was conducted while implementing a compression experiment with two different fibers with diameters of d=1.8 and 2.4 mm, using three different flexible cylinders with diameters of D=32, 42 and 74 mm, and shortening the fiber up to its elasticity limit in order to prevent plastic effects of the fiber affecting the results, which can be seen in Figs. 30–35. An analysis of the cylinder’s behavior was carried out by image processing of the video from the cylinder’s center. This analysis can indicate the amount of movement at the cylinder's center from its centerline, where the experiment began. Consequently, the mutual effect the fiber has on the cylinder and vice versa can be observed. The points (a)–(f) marked on the graph are deformation stages of the fiber and the cylinder and are visually described in the respective pictures labeled with these letters.

a

b

c

Figure 30: (a) Measured vertical force versus end shortening for fiber: and flexible cylinder: D = 32 mm, L = 500 mm. (b) Snapshots from the experiment at different levels of end shortening. End shortening is indicated by the letters a–f that appear in the force-displacement curve (a). (c) Displacement from the axis measured by image processing versus end shortening for same system.

b

a

c

Figure 31: (a) Measured vertical force versus end shortening for fiber: and flexible cylinder: D = 42 mm, L = 500 mm. (b) Snapshots from the experiment at different levels of end shortening. End shortening is indicated by the letters a–f that appear in the force-displacement curve (a). (c) Displacement from the axis measured by image processing versus end shortening for same system.

b

a

c

Figure 32: (a) Measured vertical force versus end shortening for fiber: and flexible cylinder:D = 74 mm, L = 500mm. (b) Snapshots from the experiment at different levels of end shortening. End shortening is indicated by the letters a–f that appear in the force-displacement curve (a). (c) Displacement from the axis measured by image processing versus end shortening for same system.

b

a

c

Figure 33: (a) Measured vertical force versus end shortening for fiber: and flexible cylinder:D = 32 mm, L = 500 mm. (b) Snapshots from the experiment at different levels of end shortening. End shortening is indicated by the letters a–f that appear in the force-displacement curve (a). (c) Displacement from the axis measured by image processing versus end shortening for same system.

b

a

c

Figure 34: (a) Measured vertical force versus end shortening for fiber: and flexible cylinder:. (b) Snapshots from the experiment at different levels of end shortening. End shortening is indicated by the letters a–f that appear in the force-displacement curve (a). (c) Displacement from the axis measured by image processing versus end shortening for same system.

a

b

c

Figure 35: (a) Measured vertical force versus end shortening for fiber: and flexible cylinder:. (b) Snapshots from the experiment at different levels of end shortening. End shortening is indicated by the letters a–f that appear in the force-displacement curve (a). (c) Displacement from the axis measured by image processing versus end shortening for same system.

3.2.3 Analysis of fiber and rigid cylinder behavior through fiber compression experiment

Since the flexible cylinder is not transparent, description of the point where the fiber undergoes deformation from one plane to a three-dimensional deformation can be determined both by comparing fiber behavior in a rigid transparent cylinder with a diameter of D=42 mm, and from the results of a simulation made with an ABAQUS, as presented below. For the purpose of comparing the behavior of the fiber inside a rigid cylinder to its behavior in a flexible one, as presented earlier, experiments were carried out in a rigid cylinder. These results can be seen in Figure 36. The graphs with results from the two types of cylinders are presented below.

b

a

Figure 36: (a) Measured vertical force versus end shortening for rigid cylinder: D=42 mm,L = 500 mm and   fibers: (a) d = 1.8 mm. (b).d = 2.4 mm.

3.2.4 Analysis and comparison of experiment results and image processing

Figure. 37 shows that flexible cylinders with diameters of D=42 and 74 mm exhibit similar behavior at the cylinder contact stage. In a cylinder with a diameter of D=32 mm, the cylinder adjusts itself to the fiber's (d=1.8 mm) movement and there is no rapid rise in force at the fiber's end. Therefore, there is a steep slope, as can be seen in cylinders with a diameter of D=42 and 74 mm. The highest value for force at the end of the fiber is obtained with cylinders having a D=32 mm diameter and can reach double that of other flexible cylinders. Since the cylinder diameter is smaller than the others, with greater shortenings, it resists the fiber more strongly. Therefore, there is more force at the fiber's end.

Figure 37: Measured vertical force versus end shortening for  and three different flexible cylinders: L = 500 mm, D = 32, 42 and 74 mm — compared results.

Figure 38: Measured vertical force versus end shortening for  and three different flexible cylinders: L= 500 mm, D = 32, 42 and 74 mm — compared results.

Figure 38 shows that in cylinders with diameters of D=42 and 74mm, there is similar behavior at the cylinder contact stage. In a cylinder with a diameter of D=32 mm, the cylinder adjusts itself to the fiber's (d=2.4 mm) movement. There is a moderate rise for small shortenings but there is no quick rise in force at the fiber's end. Therefore, there exists a steep slope as it appears in cylinders with a diameter of D=42 and 74 mm. The highest value for force at the end of the fiber is obtained with cylinders having a diameter, reaching up to 1.2 times that of other cylinders. Unlike experiments with fibersd=1.8 mmdiameter, here, in experiments with fibers with a diameter of d=2.4 mm, there is a greater effect on cylinders with a diameter of D=42 mm.

Figure 39: Displacement from axis measured by image processing versus end shortening for fiber: d=1.8 mm and three different flexible cylinders: L = 500 mm, D = 32, 42 and 74 mm — compared results.

Figure 39 shows the movement in the center of the cylinder as a function of the fiber’s shortening in cylinders with diameters of D=32 and 42 mm, which is very similar to that of a cylinder of diameter when experimenting with a 2.4 mm diameter fiber. That is, the movement at the cylinder center’s axial shift in a D=74 mm diameter cylinder can be up to one fifth of that of cylinders with diameters of D=32 and 42 mm.

Figure 40: Displacement from axis measured by image processing versus end shortening for fiber: and three different flexible cylinders: L = 500 mm, D = 32, 42 and 74 mm— compared results.

Figure 40 shows that the movement in the center of the cylinder as a function of the fiber’s shortening in cylinders with diameters ofD=32 and 42 mmis very similar compared to that of a cylinder of D=74 mm in experiments using a  diameter fiber. That is, the movement at the cylinder center’s axial shift in a D=74 mmdiameter cylinder, can be up to four times greater than that of cylinders with diameters of D=32 and 42 mm. Similar behavior can be observed in a fiber of d=1.8 mmdiameter.

Figure 41: Measured vertical force versus end shortening for flexible cylinder: , D= 42 mm and two different fibers:d=1.8 and 2.4 mm — compared results.

Figure 41 shows that more than double the force is required for a fiber with a diameter of d=2.4 mmto reach a similar shortening as does the d=1.8 mmdiameter fiber in experiments with flexible cylinders.

Figure 42: Measured vertical force versus end shortening for rigid cylinder: , and two different fibers: — compared results.

Figure 42 shows that more force is required for a fiber with a diameter of d=2.4 mm to

reach a similar shortening as the diameter fiber, including in experiments with rigid cylinders.

Figure 43: Measured vertical force versus end shortening for rigid and flexible cylinders: L = 500 mm, and fiber: — compared results.

Figure 43 shows that a d=1.8 mmdiameter fiber exhibits different behavior when it is within a flexible or a rigid cylinder, both at the first contact stage, and subsequently, when there are greater shortening values.

Figure 44: Measured vertical force versus end shortening for flexible and rigid cylinders: L = 500 mm, D= 42 mm and fiber:d=2.4 mm — compared results.

Figure 44 shows that a d=2.4 mmdiameter fiber exhibits greatly different behavior when it is within a flexible or a stiff cylinder, both at the first contact stage and, subsequently, when there are greater shortening values.

3.2.5 Analysis and comparison of experiment results and numerical simulation

Figure 45 shows that there is compatibility between the experiment and the numerical simulation. That is, the experimental results with a flexible cylinder can be fairly well predicted using the ABAQUS program for a d=1.8 mmdiameter fiber.

Figure 45: Measured vertical force versus end shortening for flexible cylinder: L = 500 mm, D= 42 mm and fiber:d=1.8 mm. The experimental results are compared to FE simulations results (red dashed curve).

Figure 46: Measured vertical force versus end shortening for flexible cylinder: L = 500 mm, D= 42 mm and fiber:d=2.4 mm. The experimental results are compared to FE simulations results (red dashed curve).

Figure 46 shows that compatibility exists between the experiment and the numerical simulation. That is, the experimental results with a flexible cylinder can be fairly well predicted using the ABAQUS program with a d=2.4 mmdiameter fiber.

Figure 47: Measured vertical force versus end shortening for rigid cylinder: L = 500 mm, D= 42 mm and fiber:d=1.8 mm. The experimental results are compared to FE simulations results (red dashed curve).

Figure 47 shows that there is compatibility between the experiment and the numerical simulation for stiff cylinders. That is, here too, the experimental results with a stiff cylinder can be fairly well predicted using the ABAQUS program.

Figure 48: Measured vertical force versus axial distance for flexible cylinder: L = 500 mm, D= 42 mm. The experimental results are compared to FE simulations results (red dashed curve).

Figure 48 shows a comparison of the implemented experiment on a D=42 mm cylinder with the results obtained from the numerical simulation carried out with ABAQUS, when it can be said that there is a good match between the experiment and the simulation, so that there is a good simulation infrastructure for future needs.

# Summary and conclusions

We investigated experimentally and numerically the post-buckling behavior of an elastic clamped-clamped fiber constrained inside a flexible cylinder. By using two novel experimental arrangements, which use a flexible cylinder, combined with image processing and synchronized force measurements, we studied quantitatively the evolution of contact between the fiber and the constraining cylinder. This paper presents experimental results for the evolution of deformation and contact configuration in the initial stages of deformation supported by FE simulations and analytical modeling. In addition, this study of fiber behavior considers a fiber in a flexible cylinder and includes an in-depth analysis of the fiber deformation at different loads. Various tools were used for the analysis, including representative experiments, image processing of the experimental results, finite element analysis used to simulate the experimental system, and analytical models for deformation from the onset of fiber load until after the transition to 3D deformation. As can be observed from the aggregate of simulation results in the ABAQUS software, in contrast to the experiments, hysteresis does not exist in the numerical calculation of the force on the fiber end as a function of length shortening in the ideal model of a flexible cylinder. Similarly, the force exerted by the fiber on the flexible cylinder wall increases from the moment the fiber comes into contact with the wall until reaching a maximal fixed value. The magnitude of the fiber’s force on the wall resulting from its shortening can contribute to understanding the existing mechanisms and help future applications prevent damage to the cylinder resulting from the insertion of a fiber. Additionally, because greater values of force were obtained for a fiber with d=2.4 mm in comparison to a fiber with d=1.8 mm, it can be concluded that fiber diameter affects the developing forces acting on the cylinder. The comparative graphs of the forces acting on the fiber as a function of its shortening in the two simulations in flexible and stiff cylinders show that different behavior was observed in each. These results emphasize the importance of this study’s innovation regarding flexible cylinders, and how it differs from other studies carried out on stiff cylinders (see, for example, [23]). In a similar manner, a comparable result is obtained when comparing the two experiments using flexible and stiff cylinders. Here, again, the difference in behavior of the two fiber diameters,d=1.8 mm and d=2.4 mm, is clearly observable. The existing mathematical model for this study is compatible with the experimental results, both for calculating only slight movement as well calculating larger ones. In stretching and compression experiments with the cylinders, a difference in the behavior of the flexible cylinders of various diameters was observed, which affected subsequent cylinder behavior during the various experiments. In the experiment comparing the force as a function of cylinder shortening, flexible cylinders with diameters of D=32 mm and D=42 mm were found to exhibit behavior similar to that of a flexible cylinder with a D=74 mm diameter, which then affected experimental results primarily with regard to axial movement of the cylinders. The change in distance between the cylinder’s center and the vertical axis at the experiment's start is a function of the shortening of the fiber. In the experiments, measurement of the force  as a function of fiber shortening was conducted to determine the initial contact, which is the critical point of stress causing collapse and the transition to 3D deformation. The examination of the hysteresis again revealed a dissimilarity between the different cylinders during first contact, followed by the transition to 3D deformation. The values for critical stress were comparable and a hysteresis existed for each of the cylinders. The values for the force upon the fiber ends, as a function of its shortening, were consistently large for a fiber with d=2.4 mm in comparison to a fiber with a d=1.8 mm. This result was also observed in experiments with stiff cylinders. In subsequent experiments with greater shortening between the fiber ends, the presence of a hysteresis for all cylinders and the different axial movements for various cylinder sizes could be observed. The comparative graphs of cylinders of various diameters showed that in a D=32 mm cylinder, higher force values were obtained for a fiber with d=1.8 mm. But a fiber with d=2.4 mm obtains an even greater force in a cylinder of D=42 mm. These results indicate that the forces developing during the process affected both the fiber and the cylinders. The comparative graphs of axial movement as a function of fiber shortening show that higher values were obtained with D=32 mm and D=42 mm cylinders than with a D=74 mm one, thus demonstrating the effect large cylinder diameters have on cylinder movement. Accordingly, this has implications for a number of applications. As previously mentioned, it was consistently observed that large forces were obtained from fibers with d=1.8 mm in contrast to fibers with d=2.4 mm with the same amount of shortening. As can be seen in the comparative graph, these forces can reach up to three times as much for fiber shortening in a flexible cylinder and up to four times as much in a rigid cylinder. In the last section of the presentation of experimental results, and when compared to the numerical simulation, good compatibility exists for the flexible cylinder with D=42 mm and fibers with d=1.8 mm and d=2.4 mm, and the force obtained at the fiber ends as a function of their shortening can be observed. These positive correlations demonstrate that we now have a computational tool that can be used for a variety of cylinder diameters in numerous engineering and medical applications.

The main contribution of this work is that it is now possible to characterize similar problems with a fiber in a flexible cylinder in various engineering fields and to better understand the modes of failure and how to obtain a more suitable system.

Future research should study the behavior of fibers subjected to boundary conditions different from those considered herein and extend the investigation to a range of sizes for the constraining cylinder. It is also be beneficial to use larger fiber tip displacements than those used herein to examine more complicated contact configurations. In principle, this can be done by using cylinders and/or fibers made from several types of materials and controlling their surface roughness, or perhaps by changing the fluid inside the cylinder.

Acknowledgments

This work was partially supported by the Israel Science Foundation grant ISF 581/17 to S.G.

The financial support to Y.D. by RAFAEL is gratefully acknowledged.

# References

- Lubinski, A., “A Study of the Buckling of Rotary Drilling Strings. Drilling and Production Practice,” American Petroleum Institute, Washington, D.C., 1950.

- Lubinski, A., and Blenkarn, K. A., "Buckling of Tubing in Pumping Wells, its Effects and Means for Controlling it (includes associated paper 1053-G)," 1957/1/1/ 1957.

- Seldenrath, T., and Wright, A. W., "Note on Buckling of Tubing in Pumping Wells," AIME Journal of Petroleum Transactions, vol. 213, pp. 369–399, 1958.

- Liu, J.-P., et al., "Post-buckling Analysis of a Rod Confined in a Cylindrical Tube," Journal of Applied Mechanics vol. 85(7), 2018.

- Ito, S., et al., "A Case of Fractured Guide Wire Perforating the Coronary Artery and Ascending Aorta During Percutaneous Coronary Intervention," Journal of Cardiology Cases, vol. 7, pp. e137–e141, 2013.

- Liu, C.-W., and Chen J.-S., "Effect of Coulomb Friction on the Deformation of an Elastica Constrained in a Straight Channel with Clearance," European Journal of Mechanics - A/Solids, vol. 39, pp. 50–59, 2013.

- López-Mínguez, J. R., et al., "Rupture and Intracoronary Entrapment of an Angioplasty Guidewire with the X-Sizer Thromboatherectomy Catheter During Rescue Angioplasty," Revista Española de Cardiología (English Edition), vol. 57, pp. 180–183, 2004/02/01/ 2004.

- Schneider, P. A., “Endovascular Skills: Guidewire and Catheter Skills for Endovascular Surgery,” Marcel Dekker Inc, New York, 2003.

- Katopodes, F. V., et al., "Torsional Deformation of an Endoscope Probe," Proceedings of the Royal Society of London. Series A: Mathematical, Physical and Engineering Sciences, vol. 457, p. 2491, 2001.

- Mahvash, M., and Dupont, P. E., “Stiffness Control of Surgical Continuum Manipulators,” IEEE Trans. Robot. 27, 334–345, 2011.

- Mattila, P. K., and Lappalainen, P., “Filopodia: Molecular Architecture and Cellular Functions,” Nature reviews. Molecular cell biology, vol. 9(6), pp. 446–454, 2008.

- Pronk, S., et al., "The Limits of Filopodium Stability,” Physical Review Letters, vol. 100, p. 258102, 2008.

- Leijnse, N., et al., "Helical Buckling of Actin inside Filopodia Generates Traction," Proceedings of the National Academy of Sciences, vol. 112, pp. 136–141, 2015.

- Mogilner, A., and Rubinstein, B., "The Physics of Filopodial Protrusion,” Biophysical journal, vol. 89(2), pp. 782–795, 2005.

- Tan, X. C., and Forsman, B., "Buckling of Slender String in Cylindrical Tube Under Axial Load: Experiments and Theoretical Analysis," Experimental Mechanics, vol. 35, pp. 55–60, 1995/03/01 1995.

- Liakou, A., "Constrained Buckling of Spatial Elastica Application of Optimal Control Method," Journal of applied mechanics, vol. 85(8), 2018.

- Lubinski, A., and Althouse, W. S., "Helical Buckling of Tubing Sealed in Packers," Society of Petroleum Engineers, vol. 14, pp. 655–670, 1962/6/1/ 1962.

- Mitchell, R. F., “Buckling Behavior of Well Tubing: The Packer Effect,” Soc. Pet. Eng. J. 22, 616–624, 1982.

- Cheatham, J. B., Pattillo, P.D. “Helical Post-buckling Configuration of a Weightless Column Under the Action of an Axial Load,” Soc. Pet. Eng. J. 24, 467–472, 1984.

- Paslay. P. R., and Bogy, D. B., "The Stability of a Circular Rod Laterally Constrained to be in Contact with an Inclined Circular Cylinder," Journal of Applied Mechanics, vol. 31, pp. 605–610, 1964.

- Dawson, R. and Paslay, P. R., “Drillpipe Buckling in Inclined Holes,” J. Pet. Technol. 36, 1734–1738, 1984.

- Tan, X. C., and Digby, P. J, "Buckling of Drill String Under the Action of Gravity and Axial Thrust," International Journal of Solids and Structures, vol. 30, 2675–2691, 1993/01/01/ 1993.

- Wu, J., and Juvkam-Wold, H.C, “Helical Buckling of Pipes in Extended Reach and Horizontal Wells. Part 2. Frictional drag analysis,” ASME J. Energy Resour. 115, 196–201, 1993.

- Miska, S., and Cunha, J. C., “An Analysis of Helical Buckling of Tubulars Subjected to Axial and Torsional Loading in Inclined Wellbore,” In: Proceeding SPE Production and Operations Symposium, SPE 29460, Oklahoma City, OK, April 2–4, 1995.

- Huang, N. C., and Pattillo, P.D., "Helical Buckling of a Tube in an Inclined Wellbore," International Journal of Non-Linear Mechanics, vol. 35, pp. 911–923, 2000/09/01/ 2000.

- Martinez, A., et al., "Experimental Evaluation of the Lateral Contact Force in Horizontal Wells," Journal of Energy Resources Technology, vol. 122, pp. 123–128, 2000.

- McCourt, I., et al., “Penetration of Tubulars Drill Pipes in Horizontal Oil Wells,” Proc. Inst. Mech. Eng. Part C J. Mech. Eng. Sci. 216, 1237–1245, 2002.

- Miller, J, T., et al., "Buckling of a Thin Elastic Rod Inside a Horizontal Cylindrical Constraint," Extreme Mechanics Letters, vol. 3, pp. 36–44, 2015/06/01/ 2015.

- Miller, J, T., et al., "Buckling-induced Lock-up of a Slender Rod Injected into a Horizontal Cylinder," International Journal of Solids and Structures, vol. 72, pp. 153–164, 2015/10/15/ 2015.

- Liu, J. -P., et al., “Buckling of a Slender Rod Confined in a Circular Tube: Theory, Simulation, and Experiment,” Int. J. Mech. Sci. 140, 288–305, 2018.

- Xiao, J., et al., “Three-dimensional Buckling Beam Under Cylindrical Constraint,” Int. J. Mech. Sci. 150, 348–355, 2018.

- Chen, J. -S., and C.-W. Li, "Planar Elastica Inside a Curved Tube With Clearance," International Journal of Solids and Structures, vol. 44, pp. 6173-6186, 2007.

- Wu. J, and Juvkam-Wold, H, C., “The Effect of Wellbore Curvature on Tubular Buckling and Lockup,” J. Energy Resour. Technol. 117, 214–218, 1995.

- Kuru, E., et al., “The Buckling Behavior of Pipes and its Influence on the Axial Force Transfer in Directional Wells,” ASME J. Energy Resour. 122, 129–135, 2000.

- Denoël,V., and Detournay, E., “Eulerian Formulation of Constrained Elastica,” International Journal of Solids and Structures, vol. 48, pp. 625–636, 2011.

- Timoshenko, S.P., and Gere, J. M., “Theory of Elastic Stability,” McGraw-Hill Book, 2009.

- van der Heijden, G. H. M., “The Static Deformation of a Twisted Elastic Rod Constrained to Lie on a Cylinder,” Proc. R. Soc. Lond. A 457, 695–715, 2001.

- van der Heijden, G, H, M., et al., “Spatially Complex Localisation in Twisted Elastic Rods Constrained to a Cylinder,” Int. J. Solids Struct. 39, 1863–1883, 2002.

- van der Heijden, G. H. M., “Helical Collapse of a Whirling Elastic Rod Forced to Lie on a Cylinder,” ASME J. Appl. Mech. 70, 771–774, 2003.

- Thompson, J. M. T., et al., “Helical Post-buckling of a Rod in a Cylinder: With Applications to Drill-strings,” Proc. R. Soc. A 468, 1591–1614, 2012.

- K.G. Sorenson and J.B. Cheatham Jr., “Post-buckling Behavior of a Circular Rod Constrained Within a Circular Cylinder,” ASME J. Appl. Mech. 53, 929–934, 1986.

- Chen, J.-S., and H.-C. Li, "On an Elastic Rod Inside a Slender Tube Under End Twisting Moment," Journal of Applied Mechanics, vol. 78, p. 041009, 2011.

- Chen, J.-S., and Fang, J., "Deformation Sequence of a Constrained Spatial Buckled Beam Under Edge Thrust," International Journal of Non-Linear Mechanics, vol. 55, pp. 98–101, 2013.

- Li, S.-Y., and Chen, J.-S., "A Twisted Elastica Constrained Inside a Tube," European Journal of Mechanics - A/Solids, vol. 44, pp. 61-74, 2014/03/01/ 2014.

- Fang, J., et al., "On a Compressed Spatial Elastica Constrained Inside a Tube," Acta Mechanica, vol. 224, pp. 2635–2647, 2013.

- Chen, J._S., and Chen, D.-W., "Deformation of a Spatial Elastica Constrained Inside a Springy Tube," Acta Mechanica, vol. 230, pp. 4303–4310, 2019.

- Manning. S., and Bulman, G. B., " Stability of an Elastic Rod Buckling into a Soft Wall,” Proceedings of the Royal Society, pp. 2423–2450, 2005.

- Katz, S., and Givli, S., "The Post-buckling Behavior of a Beam Constrained by Springy Walls," Journal of the Mechanics and Physics of Solids, vol. 78, pp. 443–466, 2015/05/01/ 2015.

- Katz, S., and Givli, S., "The Post-buckling Behavior of Planar Elastica Constrained by a Deformable Wall," journal of applied mechanics, vol. 84 (5), 2017.

- Vetter, R., et al., "Morphogenesis of Filaments Growing in Flexible Confinements," Computational Physics for Engineering Materials, 1–7.

- Hetenyi, M., “A General Solution for the Bending of Beams on an Elastic Foundation of Arbitrary Continuity," journal of applied physics, vol. 21, 1949.

- Jansen, J. D., “Non-linear Rotor Dynamics as Applied to Oil Well Drill String Vibrations,” J. Sound Vib. 147, 115–135, 1991.

- Rybacki, E., et al., “What Controls the Mechanical Properties of Shale Rocks? Part I: Strength and Young’s modulus,” J. Pet. Sci. Eng. 135, 702–722, 2015.

- Gholipour, A., et al., “Nonlinear Biomechanics of Bifurcated Atherosclerotic Coronary arteries,” Int. J. Eng. Sci. 133, 60–83, 2018.

- Craiem, D., et al., “Coronary Arteries Simplified with 3D Cylinders to Assess True Bifurcation Angles in Atherosclerotic Patients,” Cardiovassc. Eng. 9, 127–133, 2009.

- Hirai, T., et al., “Stiffness of Systemic Arteries in Patients with Myocardial-infarction–a Non-invasive Method to Predict Severity of Coronary Atherosclerosis,” Circulation 80, 78–86, 1989.

- Tran, J. S., et al., “Uncertainty Quantification of Simulated Biomechanical Stimuli in Coronary Artery Bypass Grafts,” Comput. Methods Appl. Mech. Eng. 345, 402–428, 2019.

- Chen, J.-S., and Wang, L.-C., “Contact Between Two Planar Buckled Beams Pushed Together Transversely,” International Journal of Solids and Structures, vol. 199, 181–189, 2020.

- Chai, H., "The Post-buckling Response of a Bi-laterally Constrained Column," Journal of the Mechanics and Physics of Solids, vol. 46, pp. 1155–1181, 1998/07/01/ 1998.

- Timoshenko, S., “Strength of Materials,” D. Van Nostrand Company. Inc, 1948.

- Frisch-Fay, R., “Flexible Bars”, Butterworth & Co. Publishers, 1962.