The information contained in this document is confidential, privileged and only for the information of the intended recipient and may not be used, published or redistributed.

Editing Sample – Environmental Sciences

Passive sampling for ozone exposure: A case study of southeastern Hyogo prefecture, Japan

Keywords: Nitrate; ozone; passive sampler; personal exposure; southeastern Hyogo prefecture

Highlights:

- Passive ozone samplers used to study ozone exposure in southeastern Hyogo prefecture

- Association of outdoor ozone concentration and from AMS investigated

- Mechanism of estimation is based on oxidation of nitrite by ozone to produce nitrate

Ozone is a powerful oxidant and a secondary air pollutant formed by photochemical interactions of nitrogen oxides and reactive volatile organic compounds. Acute and chronic effects of ozone exposure on the respiratory system have been investigated in several epidemiological studies.1-3) Researchers have also reported the negative effects of ozone on cardiovascular and respiratory mortality.4-6) A strong relationship was reported between the measured ozone concentration in outdoor workers and the ambient ozone levels obtained from the Ambient Monitoring Station (AMS) in proximity,7), and the concentrations of ozone were usually the highest and the lowest in the outdoor and indoor environment, respectively.8, 9) Therefore, the data from the AMS was occasionally used to evaluate the health risk of ozone.5, 6) As the ozone is formed primarily in the atmosphere, the ambient conditions and the wind direction surrounding the AMS are likely to affect the regional representativeness in evaluating ozone levels. As an alternative, the passive ozone sampler 10) is a useful tool to accurately measure the personal ozone exposure.8, 9, 11, 12) with an imprecision of ±4 ppb and a relative imprecision of 10%.13)

In this study, passive ozone samplers were used to evaluate the ozone exposure levels of the staff of Hyogo College of Medicine, Japan. Simultaneously, the outdoor ozone concentrations were also determined. This is the first study to detect the personal exposure level of residents from southeastern Hyogo prefecture to ozone. This study also investigated the association of the outdoor ozone concentrations and that from the AMS in proximity to the outdoor sampler.

MATERIALS AND METHODS

Sampling and Pretreatment

Twenty-one staff members of the Hyogo College of Medicine, Japan were recruited as study participants. This study was approved by the Ethics Committee of Hyogo College of Medicine, and informed consent was obtained from each participant before the study. The houses of all participating staff were located in five cities of southeastern Hyogo prefecture (Fig. 1) and had a set of personal/outdoor passive ozone samplers (Ogawa sampler, OGAWA & CO., LTD., Kobe, Japan). The personal sampler was pinned to the top front side of participant's clothes. The outdoor sampler was placed in a well-ventilated area outside of the participant's house. A total of forty-two 24-h samples were collected and stored in a refrigerator (4 ˚C) until use. The participants were instructed to record their daily activities, such as the time spent outdoors for 07:00–12:00, 12:00–18:00, and 18:00–24:00.

The sampler used in this study contains two glass fiber filters coated with sodium nitrite and potassium carbonate.14) The mechanism is based on the oxidation of nitrite by ozone to produce nitrate. In this study, the filters were treated according to the Ozone Passive Sampler Protocol published by OGAWA & CO., LTD. 14) Two glass fiber filters were placed in a vial, and subsequently, 5 mL of ultra-pure water (Milli-Q) was added. After shaking, the vial was left to stand for approximately 30 min. Approximately 1 mL of the supernatant was filtered (Cosmonice Filter (W), pore size 0.45 μm, φ13 mm, Nacalai Tesque, Kyoto, Japan), and 100 µL of the filtrate was injected into an HPLC system as described below.

HPLC system and Chemicals

Nitrate was analyzed using an HPLC system with ultraviolet (UV) detection. The HPLC system consisted of a mobile phase pump (SI-1/2001, Shiseido, Tokyo, Japan), an UV-visible detector (SI-1/2002, Shiseido, Tokyo, Japan), a chromatopac integrator (C-R7A, Shimadzu, Kyoto, Japan), a degasser (SI-1/2009, Shiseido, Tokyo, Japan), a column oven with manual injector (SI-1/2004, Shiseido, Tokyo, Japan), a guard column (Shim-pack IC-GA2, 4.6 ID × 10 mm, Shimadzu, Kyoto, Japan) and a non-suppressor type analytical column (Shim-pack IC-A3, 4.6 ID × 150 mm, Shimadzu, Kyoto, Japan). The mobile phase was 0.4 mM disodium phthalate (Tokyo Kasei, Tokyo, Japan), and flow rate was 1.0 min/mL. Both the guard and analytical columns were maintained at 40ºC in the column oven. Standard nitrate (sodium nitrate) was purchased from Kishida Chemical Co., Ltd. (Osaka, Japan).

Quality Control

Indirect photometric detection ion chromatography was used in this study for nitrate analysis. Standard nitrate solution (100 µL) was injected into the analysis system for method validation. The calibration curve showed good linearity (r = 0.9933) from 0.1 to 1.0 µg/mL. The detection limit (S/N = 3) was 0.1 µg/mL, and relative standard deviation (n = 3) was 4%.

In order to assess the accuracy of the sampler, data from the samplers were compared to that from UV Ozone Monitor (EG-700, EBARAJITSUGYO Co., Ltd., Kanagawa, Japan) at the roof of No. 9 building of Hyogo College of Medicine. The mean nitrate concentrations of five 24-h samples obtained from passive sampler and UV Ozone Monitor were 23.5 (± 2.96) ppb and 23.9 (± 2.14) ppb, respectively. The ([UV Ozone Monitor] - [passive sampler]) / [UV Ozone Monitor] ratio of each data was within 10% (range from 0.9 to 8.4%). Approximately 10% of the samples from the same location were used as field blanks. The field blank values were subtracted from the ozone measurement for final calculation of the nitrate concentration.

Data Analysis

For statistical treatment, correlation and multiple regression analyses were performed using a statistical analysis program (PASW Statistic 18, IBM, U.S.A.). For multiple regression analysis, the model form was as follows:

where Co and Ca are the outdoor and AMS ozone concentrations, respectively; P is the relative position (of the outdoor sampler to the nearest AMS based on the wind direction. It includes leeward, windward and parallelism); D is the distance (between the outdoor sampler and its nearest AMS) (from 0.5 to 4.4 cm), and ε is the error term.

The information contained in this document is confidential, privileged and only for the information of the intended recipient and may not be used, published or redistributed.

Proofreading Sample

Marine biopolymer-based nanomaterials as a novel platform for theranostic applications

Abstract

Marine biopolymer-based nanomaterials are one of the most active research areas in recent decades for theranostics applications. Marine biopolymers are interesting biomaterials for clinical applications because of their good biocompatibility, biodegradability, inexpensiveness, abundance, stability, ease of surface modification, and nontoxicity. During development, new nanoparticles are coated with marine polymers to combine therapeutic and diagnostic (theranostic) applications because of the strongly enhanced absorption and scattering in near-infrared (NIR) regions. In this review article, the use of marine biopolymer-based nanomaterials for theranostic applications is evaluated, addressing potential applications in drug delivery, photothermal therapy (PTT), photodynamic therapy (PDT), hyperthermia therapy, photoacoustic imaging (PAI), magnetic resonance imaging (MRI), and computed tomography (CT). In addition, the most recent progress in the biocompatibility of marine biopolymer-based nanomaterials in vitro and in vivo is discussed, along with a promising future scope for the treatment of major life-threatening diseases, such as cancer.

Keywords: Marine biopolymer, Nanomaterials, Theranostic applications, Cancer, Near-infrared.

Introduction

Marine biopolymer-based nanomaterials have attracted much attention in the latest decades for theranostic applications, including drug delivery, photothermal therapy (PTT), photodynamic therapy (PDT), hyperthermia therapy, photoacoustic imaging (PAI), magnetic resonance imaging (MRI), and computer tomography (CT) [1-3]. Marine organisms are known as the rich sources of polymers, including fucoidan, alginate, carrageenan, porphyran, chitosan, and chitosan oligosaccharides. Marine biopolymers are one of the main constituents of all living organisms and have been receiving attention as interesting biomaterials in recent years for the medical field because of their good biocompatibility, biodegradability, inexpensiveness, abundance, stability, ease of surface modification, and nontoxicity [2, 4]. In the past decade, inorganic nanoparticles such as gold (Au), silver (Ag), magnetic, mesoporous silica, copper sulfide, and upconversion nanoparticles have been coated with marine polymers and have opened several new avenues for these nanomaterials for biomedical applications because of their strongly enhanced absorption in NIR regions [5].

Cancer is one of the main public problems worldwide [6] and is one of the major problems for biomedical research in recent times [7]. Presently, cancer treatment commonly relies on radiotherapy and chemotherapy and is considered problematic to treat because of the very limited available treatment options, difficulty in receiving timely diagnosis, and various side effects [8]. In this context, considerable effort has been dedicated on discovering new strategy for diagnostic and therapeutic applications. In recent years, the development of novel nanomedicine has made a great breakthrough in the biomedical fields [9]. A wide range of marine biopolymer-based nanomaterials have been developed and used for theranostic applications. This review focuses on the marine biopolymer-based nanomaterials that prove to be an important tool for several theranostic applications, including drug delivery, photoablation therapy, imaging and treatment of major life threatening diseases.

Marine biopolymer

Marine biopolymers are one of the most abundant, naturally occurring polymers produced by all living organisms. Biopolymers are long chains of carbohydrate molecules of repeating monosaccharide units combined together by glycosidic bonds [1]. Marine biopolymers have various sources from marine algae and crustaceans. Fucoidan, alginate, carrageenan, and porphyran are natural biopolymers obtained from seaweeds. Chitosan and chitosan oligosaccharide are natural biopolymer isolated from marine crustaceans [2]. Marine biopolymers are naturally occurring polymers with storage and structural functions, which are one of the major constituents in biosystems, including the glycocalyx and the extracellular matrices. Marine biopolymers are renewable, stable, cheap, abundant, biocompatible, biodegradable, and nontoxic in nature. In recent years, biopolymer have commonly been used for various applications in cosmeceuticals, nutraceuticals, and pharmaceuticals [10].