- Self-diagnosis, electrical testing

- Self-diagnosis of the air conditioning system

Starting in model year 2008, a manually controlled air-conditioning system will be offered for the XCD X5 in addition to the automatically controlled system. Depending on the software in the different control units connected to the data bus system, errors can be displayed in the manually controlled air-conditioning system such as the error message, "Please query error memory of the control unit for the

Climatronic H255“. With vehicles that have a manually controlled air-conditioning system, read out the error memory of the control unit for the H301 control unit.

- Error detection

When malfunctions arise in the monitored sensors or components, they are saved in the error memory of the respective control unit, and the type of error is indicated. If the error stops for a certain period, it becomes a sporadic error.

If the specified values are not reached in display group "003" for automatically controlled air-conditioning systems even though the system’s cooling output is satisfactory (for example because of unfavorable environmental conditions such as an excessively high surrounding temperature), adjust the speed of the engine to approximately 2,000 RPM before initiating the default setting, and initiate the default setting at this engine speed.

Description of the compressor shut-off condition in display group 002

At present, only shut off conditions that have an * in the following table are entered in display fields 2, 3 and 4. However, an entry is only made when the shut-off condition exists for more than 20 seconds (shutoff condition 12 longer than 12 seconds). Depending on the design of the control and display unit (software version), the compressor shut-off conditions in display fields 2-4 are not always deleted when the error memory is deleted. You therefore always need to note the associated entry in the error memory.

- Sicherheitsmaßnahmen

Für diese Kältemittel sind folgende Sicherheitsmaßnahmen zu beachten (in einzelnen Ländern können zusätzliche Vorschriften gelten). Ist es bei Reparaturarbeiten notwendig, den Kältemittelkreislauf zu öffnen, zuerst Kältemittelkreislauf entleeren. Dabei ist jede Berührung mit flüssigem Kältemittel oder Kältemitteldämpfen zu vermeiden. Sollte trotz Beachtung aller Sicherheitsmaßnahmen Kältemittel austreten, darf das entstehende Kältemittel-/Luftgemisch nicht eingeatmet werden. Deshalb vorhandene Absauganlagen einschalten, Hände mit Gummihandschuhen und Augen mit Schutzbrille schützen.

- Schalttafelausströmer und Luftführungen

Schalttafel - mit den Luftführungen zu den Defrosterdüsen und den Schalttafelausströmern Fahrer- und Beifahrerseite

Defrosterdüse

Für Frontscheibe

Seitenscheibe/ Beifahrerseite

Seitenscheibe/ Fahrerseite

Wurde beim Einbauen der Schalttafel das Zwischenstück zwischen Heizung und Luftführungskanal in der Schalttafel gequetscht, ist die Luftführung zur Frontscheibe ungleichmäßig oder ungenügend.